- University of Naples

- Flight Simulator

- User Manual

- Table of Contents

Table of Contents	2

1	Introduction	5

1.1	Document Purpose	5

1.2	History of this document	5

1.3	Abbreviations used in this document	5

2	Hardware overview	6

2.1	Simulator area	6

2.1.1	Briefing room	7

2.1.2	Supervisor room	7

2.1.3	Main simulation room	7

2.2	Computer distribution	7

2.3	Power on all equipments	9

2.4	Power off all equipments	9

3	Software overview	10

3.1	Module distribution	10

3.2	Modules description	11

3.2.1	Visuals	11

3.2.2	Flight instruments	11

3.2.3	Console Acquisition	11

3.2.4	Dynamic model	11

3.2.5	Sound	11

3.2.6	Simulation user interface	12

3.2.7	Security	18

3.2.8	Motion	18

3.3	Start simulation	19

3.4	Pause / Run simulation	20

3.5	Stop simulation	21

3.6	Network exchanges	22

3.6.1	Wizard Server (fgrun) and Security server	22

3.6.2	Flight Dynamic Model	22

3.6.3	Console Acquisition	23

3.6.4	Motion Platform	23

4	Configuration files	24

4.1	PFC configuration file	24

4.1.1	Tags description	24

4.1.2	Defining a PFC control	25

4.1.3	Default PFC controls assignation	29

4.1.4	Console Acquisition allowed properties	30

4.2	Modules configuration file	31

4.2.1	Tags description	31

4.2.2	Defining a module	32

4.3	Force Feedback Configuration files	35

4.3.1	FFB main configuration file	35

4.3.2	FFB configuration tables	36

4.4	ViewCorrection Configuration file	37

4.4.1	Tags description	38

4.5	Security Configuration file	39

4.5.1	Tags description	39

4.5.2	Defining a module	40

4.6	DAQ IO Configuration file	41

5	Precision Flight Console	42

5.1	Controls description	42

5.2	Data send by PFC console	43

5.3	Controls state when simulation starts	44

5.4	Console Acquisition Mapping	45

6	Console Acquisition parameters	46

7	Record / Replay	47

7.1	Record / Replay functionality	47

7.2	Record output files formats	48

7.2.1	“Initial condition” file format	48

7.2.2	“Output” file format	49

7.2.3	“Input output” file format	50

7.3	Replay input file format description	51

8	Force Feedback	52

8.1	Configuration files	52

8.2	Coordinate System	53

8.2.1	JSBSim (body axes)	53

8.2.2	FlightGear (aircraft model)	53

8.3	Equations for Elevator	55

8.3.1	Gearing ratio	56

8.3.2	Trim position	57

8.3.3	Inertial coupling actions	58

8.3.4	Aerodynamic hinge moment	61

8.4	Equations for Ailerons	64

8.4.1	Aileron Gearing ratio	64

8.4.2	Aileron Trim position	65

8.4.3	Inertial coupling actions	66

8.4.4	Aerodynamic hinge moment	67

8.5	Equations for Rudders	69

8.5.1	Gearing ratio	70

8.5.2	Trim position	71

8.5.3	Inertial coupling actions	72

8.5.4	Aerodynamic hinge moment	73

9	Compilation	76

9.1	Module description and libraries dependence	76

9.1.1	RExecD	76

9.1.2	TerraGear	76

9.1.3	Scenery Designer	76

9.1.4	Atlas	76

9.1.5	Console Acquisition	77

9.1.6	Fgrun (wizard flight gear)	77

9.1.7	FlightGear	77

9.1.8	Security PTF	77

9.1.9	Motion Interface	77

9.2	Modules / Libraries under Windows	78

9.3	Modules / Libraries under Linux	78

9.3.1	PLIB	78

9.3.2	ZLIB	78

9.3.3	SIMGEAR	78

9.3.4	FLIGHTGEAR	80

9.3.5	FLTK	80

9.3.6	FGRUN	80

9.3.7	ATLAS	80

9.3.8	FLU	80

9.3.9	GLIB	80

9.3.10	LPNG	80

9.3.11	GTS	82

9.3.12	JPEG	82

9.3.13	NURBS++	82

9.3.14	TerraGear	82

9.3.15	fgsd (Scenery DESIGNER)	82

10	Source code modifications	83

10.1	FlightGear	83

10.1.1	New protocols	83

10.1.2	New options	83

10.1.3	Shared memory	84

10.1.4	“Watch dog” support	84

10.1.5	Change to the Windows Visual project	84

10.2	Fgrun	84

10.3	SimGear	84

10.4	Atlas	84

11	Annexes	85

11.1	JSBSim properties	85

11.2	Flight Console Protocol	90

# Introduction

## Document Purpose

This document describes the entire flight simulator platform, from hardware architecture to configurations files needed in simulation, or source code changes in FlightGear.

## History of this document

| Version | Date | Writer(s) | Differences from previous release |  |
| --- | --- | --- | --- | --- |
| 1.0 | 11/02/2005 | Dominique DA GRAÇA | Creation |  |
| 2.0 | 22/12/2005 | Agostino De Marco | Force Feedback modifications |  |
| 3.0 | 28/03/2006 | Dominique DA GRAÇA | Final version |  |
| 3.1 | 23/08/2006 | Dominique DA GRAÇA, Jean-Michel HESPERT | Minor corrections Chapter about Record/ Replay added |  |
| 3.2 | 27/09/2006 | Jean-Michel HESPERT | §4.3.2: warning concerning the fact that the values given by the force/voltage conversion tables for the ailerons are the forces applied at the position of the pilot hand on the yoke. |  |

## Abbreviations used in this document

| FG | FlightGear |
| --- | --- |
| PFC | Precision Flight Console |
| PTF | Platform |
| PC | Personal computer |
| FDM | Flight Dynamic Model |
| NI | National Instrument |

# Hardware overview

## Simulator area

The simulator area is divided in three rooms:

- Briefing room

- Supervisor room

- Main simulation room

### Briefing room

There is no simulation hardware in this room.

A warning light is put on top of the door (between this room and the supervisor room) to indicate if a simulation is currently running or not.

### Supervisor room

The supervisor room contains the set of seven computers, control television, microphones, and sound amplifier needed to run the simulatior.

This room is equipped with an emergency button and a sensor put on top of the simulation’s door. These equipments set electric power off in the simulation room.

### Main simulation room

This room contains the flight cockpit set on the “hexapod” motion platform, the stairs to access cabin, the white screens with projector, and few computers.

In this room, the security is provided by different sensors put on:

- The truck access door

- The external access door

- The stairs parking place

- The cockpit door

An emergency button is also present in the aircraft cockpit.

These equipments set electric power off in the simulation room.

## Computer distribution

Eight computers (PC) are used to efficiently run the flight simulation.

Six PC are set in the supervisor room, one other is embedded in the aircraft cockpit, and the last one is put near the motion platform control box.

Notice that another PC is used to run the Scenery Designer and Terra Gear software.

But this computer is not directly used to run the simulation.

We actually use Windows 2000 for the Motion PC, Windows XP for SUNEMBEDED, SUNCTRL3 and SUNCTRL4. Other PC’s are under Linux Red Hat Fedora 4.

Main simulation room

Aircraft cockpit (on hexapod)

Atlas / Instruments / FFB

SUNEMBEDED

Hexapod

SUNPTF

Motion platform

Supervisor room

SUNVISU1

SUNVISU2

SUNVISU3

3D visual

Dynamic aircraft model

SUNCTRL2

fgrun, Security

SUNCTRL4

Sound

SUNCTRL3

Scenery Designer

Terra Gear

SUNCTRL1

The computers rack in the supervisor room is composed as shown is the following diagram:

Computer Rack

SUNVISU3

SUNVISU2

SUNVISU1

SUNCTRL4

SUNCTRL3

SUNCTRL2

SUNCTRL1

## Power on all equipments

To start all the equipments for a simulation, you need to first start SUNCTRL4. When the PC is logged in, you can start all other PC’s, the motion platform, and the sound equipment.

## Power off all equipments

To stop all the equipments, turn off the sound equipment. Then, there is a shortcut on SUNCTRL4 that send a stop order to all PC’s. You then have to turn off the motion platform.

# Software overview

## Module distribution

Modules are software programs that run on a host computer.

Here is the complete list of modules used in the aircraft simulation:

| Computers | Modules | Description |
| --- | --- | --- |
| SUNVISU1 | Left Visual | Displays left screen part (FG). |
| SUNVISU2 | Centre Visual | Displays middle screen part (FG). |
| SUNVISU3 | Right Visual | Displays right screen part (FG). |
| SUNEMBEDED | Flight Instruments | Displays flight instruments (FG). |
| SUNEMBEDED | Console Acquisition | Reads input data from PFC and manages inputs/outputs of force feedback yoke and rudders. |
| SUNEMBEDED | Atlas | Displays charts of world scenery used in FG. |
| SUNCTRL2 | Aircraft dynamic Model Simulator | Dynamic aircraft model (FG). |
| SUNCTRL3 | Sound | Sound module. |
| SUNCTRL4 | Main simulation user interface (fgrun) | User interface for supervisor (select aircraft, Start, Stop and pause the simulation…). |
| SUNCTRL4 | Simulation Security | Checks modules on simulation for security |
| SUNPTF | Motion | Management of motion platform. |

- These modules are not used during simulation.

| Computers | Modules | Description |
| --- | --- | --- |
| SUNCTRL1 | Terra Gear | Convert GIS data in 3d, and generate 3d data (airport, lake…). |
| SUNCTRL1 | Scenery Designer | Design custom sceneries |

## Modules description

### Visuals

The visuals are based on FlightGear software.

They are in charge of computing and displaying 3d data.

Retro projectors are used to display the 3 images on the white screens. The graphic resolution applied is 1400x1050 pixels.

The flight dynamic model is performed by another module on another computer. So these modules are configured to be run with external FDM.

- The left and right visuals offer a 60 degrees heading view offset from central visual.

### Flight instruments

The instruments are based on FlightGear.

They are in charge of displaying 2d instruments on the tactile screen embedded in the cockpit.

As the visuals, the FDM is external.

### Console Acquisition

The Console Acquisition is the main module in charge of reading inputs data from pilot actions and producing the force feedback sensation to aircraft yoke, and rudders commands.

This module is decomposed in two main parts:

- It reads data from the Precision Flight Console (such as flaps, parking brakes…), and returns information to the flight dynamic model

- It moves the different motors (aileron, elevator and rudders) according to pilot’s effort and then computes the force feedback with the flight dynamic model

### Dynamic model

The dynamic model is based on FlightGear.

The JSBSim is used in the simulation.

3d display and sound are disabled.

### Sound

- The sound is also based on FG. 3d display is disabled for this module.

### Simulation user interface

This module is based on fgrun (from FlightGear).

This is the interface that allow user to select the aircraft, the location and to configure FlightGear.

Another page was added to this interface to run, pause or stop the simulation.

The user interface is composed of five different pages:

#### Directory and File configuration page

- This page is used to configure Flight Gear paths and the module file for the simulation.

| Field name | Description |
| --- | --- |
| Executable | This field contains the FG executable filename. |
| FG_ROOT | This field indicates the FG data path. |
| FG_SCENERY | This field indicates the FG scenery path. |
| Airports Cache | FG automatically generates this file. |
| Module Configuration file | This file contains a description of the modules used for the simulation. |

#### Aircraft page

This page is used to select the aircraft used in the simulation.

Currently, only the C172p aircraft have the force feedback properties set for the simulation.

#### Airport page

- This page is used to select the airport and runway.

Remark: Take care to select an airport.

If you click on the “next” button too quickly, then, no airport is selected. It will produce an error later when starting the simulation.

#### Advanced options page

This page resumes the FG options actually used on simulation.

A complete description of these options is provided on:

http://www.flightgear.org/Docs/InstallGuide/getstart.html
(or the getstart.pdf in the FG directory)

- Remark:

Some options will have no effect in the simulation.

When the visuals are launched (when you start the simulation), they take a list of parameters from the Module.xml configuration file, which can replace options you have selected in this page. Naturally, you can change these options by modifying the configuration file.

#### Simulation page

This page is used to run, pause or stop the simulation.

The flight simulation is composed of different modules distributed on different computers.

Each line of the tab shows a module. The state of the module is displayed on the right column (the module can be Off, Loading, Running, in Pause, or in connection error).

- A white line mean that the module is enable

- A gray line mean that the module is disabled

- If an error occurred, the line will be displayed in orange (network error).

IMPORTANT NOTE

When the simulation page is displayed after pressing “next button”, the module.xml configuration file is loaded.

If you modify this file while the simulation page is displayed, you will need to press “previous button” to go back into options page, then press again “next button” to return into simulation page. This will reload the module.xml configuration file, and your modifications will take effect.

### Security

The security is a module that checks the main modules when the simulation is running.

If an important module, such as the dynamic model, is not responding or is shut down, then, the security module will immediately stop the simulation.

- The blue lines represent the physical sensors (on the top of doors) or the emergency buttons.

- The gray lines show each module currently connected to the security module.

- The white lines show modules that are under supervision of the security module.
In the picture below, if the dynamic model is shut down or is not responding, then, the security module will send an emergency stop.

### Motion

The motion module manages the hexapod (motion platform) according to the dynamic model data (aircraft accelerations). This module is based on CueSim library.

## Start simulation

To start a simulation, you have to double click on the FlightSim shortcut on the desktop.

This launches the FG user interface (FlightGear Wizard).

Then, you can select your plane, airport and options.

To the simulation page, you only have to click on start button to launch the simulation.

Server error:

When you click on the next button in the option page, you activate the simulation page.

When this page is activated, all the daemon servers on distant computer are checked.

If a computer is not responding (or if the server is down), then an error occurred, and the name of the faulty PC is displayed.

You have to check computer manually.

## Pause / Run simulation

When all modules are launched correctly, they start in pause state.

- Then, by clicking on run button, all modules will start running.

If one or more module is in error state, the simulation cannot be run.

In this case, you need to stop all, and restart again.

## Stop simulation

- The stop button has two states.

- When clicked for the first time, it sends a stop message to all running modules.

When error occurs (such as network error, or a killed module), the stop order will take time to be processed (it waits for a module acknowledgment for few seconds).

- By clicking a second time on the stop button, the wizard will immediately kill all modules that are not responding.

## Network exchanges

Here is a description of network exchanges between modules.

It is very important to undestand how modules are linked together during simulation.

There is two connexions type:

- Classic TCP/IP

- FlightGear Protocol (UDP)

### Wizard Server (fgrun) and Security server

All modules lauched by the FlightGear Wizard (fgrun) are connected to the:

- Wizard server

- Security server

- These connexions are in TCP/IP protocol.

The connexion to the wizard server is used to send module’s following orders:

- Run

- Pause

- Stop

The connexion to the security server is used to send module’s following orders:

- IsAlive (check module)

- ModuleName (Ask for module’s name)

- EmergencyStop (Set module to pause state)

### Flight Dynamic Model

| Source Module | Protocol | Destination Module |
| --- | --- | --- |
| Flight Dynamic Model | FFBinputs | Console Acquisition |
|  | Nmea | Atlas |
|  | NativeFDM | Left Visual |
|  |  | Center Visual |
|  |  | Right Visual |
|  |  | Motion |
|  |  | Instruments |
|  |  | Sound |

### Console Acquisition

| Source Module | FG Protocol | Destination Module |
| --- | --- | --- |
| Console Acquisition | FFBoutputs | Flight Dynamic Model |
|  | PFC-CTRLS | Flight Dynamic Model |
|  |  | Instruments |

### Motion Platform

| Source Module | FG Protocol | Destination Module |
| --- | --- | --- |
| Motion Platform | View-Correction | Left Visual |
|  |  | Center Visual |
|  |  | Right Visual |

# Configuration files

The simulator use xml configuration files.

Here is a complete description of these files.

## PFC configuration file

The Console Acquisition module uses this file.

The PFC configuration file is used to map hardware controls to Flight Gear internal properties.

It is also used to calibrate analogical controls.

### Tags description

#### <ControlList>

The <ControlList> tag only contains the description of all the PFC controls.

#### <control>

The <control> tag is used to define a PFC control.

#### <code>

This tag contains the hexadecimal control code send by the console (each controls have a specific code).

#### <desc>

String used for a description of the control command. It is useful for the debug mode.

#### <calibrate>

This tag defines the PFC calibration (used only on analog controls).

#### <min_theoric>, <middle_theoric>, <max_theoric>

These tags defines the theoretical values returned by the control (for calibration).

#### <min_real>, <middle_real>, <max_real>

These tags defines the real values returned by the console (for calibration).

#### <state1>, <state2>, … <state6>

These tags defines flight gear properties that are assigned to the control.

Only the properties defined in the Console Acquisition Software are valid.

#### <debug>

This tag is very useful for debugging control assignment. It prints on the screen command name, property assigned with the command, and corresponding hexadecimal value.

### Defining a PFC control

A PFC control description looks like:

<control>

<code>0x80</code>

<desc>Ailerons</desc>

<calibrate>

- <min_theoric>0x00</min_theoric>

<middle_theoric>0x40</middle_theoric>

<max_theoric>0x7F</max_theoric>

<min_real>0x09</min_real>

- <middle_real>0x37</middle_real>

<max_real>0x70</max_real>

</calibrate>

<state1>

<property>/controls/flight/aileron</property>

</state1>

<debug>no</debug>

</control>

A control is defined by a code <code>, a description <desc>, an optional calibration for analog controls <calibrate>, and 1 to 6 states <state1…6>, and an optional debug tag <debug>.

#### <code>

Syntaxe : <code>0x00</code>

The code tag, is a 2 hexadecimal number that correspond to the first command byte send by the PFC console. Each controls have a specific code (see the flight console protocol).

#### <desc>

Syntaxe : <desc>A simple text</desc>

- The description tag, is needed to easily debug control command.

#### <calibrate>

Syntax :

<calibrate>

<min_theoric>0x00</min_theoric>

<middle_theoric>0x40</middle_theoric>	Optional

<max_theoric>0x7F</max_theoric>

<min_real>0x09</min_real>

<middle_real>0x37</middle_real>		Optional

<max_real>0x70</max_real>

</calibrate>

The theoretical values are defined by PFC (see flight console protocol) for each command and should not be changed.

The real values are read directly from console (it depends on the control device

hardware).

#### <state1…6>

Syntax :

<state1>

<property>/controls/flight/aileron</property>

…

</state1>

The state tag defines 0, 1 or more properties to be applied when de command is received by the console acquisition software. The property must correspond to an existing Flight Gear data.

There are 5 property syntaxes:

##### Default data assignation

- The default assignation is done by

<property>/path/data</property>

The value assigned to /path/data is the value returned by the PFC console.

If the <calibrated> tag is present, the value return by PFC is calibrated (and clamped) before being assigned to data.

The Console Acquisition Software only performs calibration on following controls :

- Throttle Left / Right

- Propeller pitch Left / Right

- Mixture Left ./ Right

- Yoke Elevators and Ailerons

- Rudder Trim

- The Console Acquisition Software limits

floating values to [–1.0,1.0] and

integers values to [–1000, 1000].

##### Direct assignation value

- The direct assignation value is done by

<property>/path/data=1.0</property>

- The number given is a always a floating value.

##### Direct increase or decrease value

<property>/path/data+=0.2</property>

<property>/path/data-=0.2</property>

The number given is a always a floating value.

The /path/data will be increases or decreases by given value.

##### Boolean toggle

<property>/path/data=bool_toggle</property>

- The /path/data value will toggle alternatively when control command is pushed (0.0 or 1.0).

##### Special assignation

- The %d in

/controls/fuel/tank[%d]/boost-pump[0]=0

is hardly assigned to

/controls/fuel/tank[0]/fuel_selector

This only applies to fuel boost left and right.

This can be changed in the future by added a <link> tag.

#### <debug>

Syntax : <debug>yes/no</debug>

It prints on the screen the command name, the first byte command (control byte) and the second byte command (the value byte), if there is one.
The property name and it’s value are also printed if defined in the state tag.
If a property is defined but doesn’t appear on screen, it generally means that property is not defined in Flight Gear (this is usually due to a syntax error).

### Default PFC controls assignation

The table below shows how is mapped PFC controls to flight gear data (default XML file mapping).

| Controls | Description |
| --- | --- |
| Magneto Left | /controls/engines/engine[0]/magnetos /controls/engines/engine[0]/starter (when OFF) |
| Magneto Right | /controls/engines/engine[1]/magnetos /controls/engines/engine[1]/starter (when OFF) |
| Engine Start | /controls/engines/engine[0]/starter (left) /controls/engines/engine[1]/starter (right) |
| Navigation lights | /controls/lighting/nav-lights |
| Strobe lights | /controls/lighting/strobe |
| Landing lights | /controls/lighting/landing-lights |
| Taxi lights | /controls/lighting/taxi-light |
| Parking Brake | /controls/gear/brake-parking |
| Battery master | /controls/engines/engine[0]/master-bat /controls/engines/engine[1]/master-bat |
| Left alternator | /controls/engines/engine[0]/master-al |
| Right alternator | /controls/engines/engine[1]/master-al |
| Avionics master | Not used (PFC controls not send at all) |
| Throttle left | /controls/engines/engine[0]/throttle |
| Throttle right | /controls/engines/engine[1]/throttle |
| Propeller pitch Left | /controls/engines/engine[0]/propeller-pitch |
| Propeller pitch Right | /controls/engines/engine[1]/propeller-pitch |
| Left Mixture | /controls/engines/engine[0]/mixture |
| Right Mixture | /controls/engines/engine[1]/mixture |
| Flaps | /controls/flight/flaps |
| Cowl flaps | /controls/flight/aileron-trim |
| Landing gear | /controls/gear/gear-down |
| Alternate air | Not assigned to FG data: used to trigger the “record” function. |
| Rudder trim | /controls/flight/rudder-trim |
| Tank select | /controls/fuel/tank[0]/fuel_selector /controls/fuel/tank[1]/fuel_selector /controls/engines/engine[0]/starter (when OFF) /controls/engines/engine[1]/starter (when OFF) |
| Fuel boost left | /controls/fuel/tank[selected_tank]/boost-pump[0] |
| Fuel boost right | /controls/fuel/tank[selected_tank]/boost-pump[1] |
| Pitot heat | Not assigned to FG data: used to trigger the “replay” function. |
| Yoke | /controls/flight/aileron /controls/flight/elevator |
| CH controllers on yoke | /controls/flight/elevator-trim |

### Console Acquisition allowed properties

The console acquisition software is restricted to hardly defined flight gear properties.

These properties are defined below.

/controls/flight/aileron

/controls/flight/aileron-trim

/controls/flight/elevator

/controls/flight/elevator-trim

/controls/flight/rudder

/controls/flight/rudder-trim

/controls/flight/flaps

/systems/electrical/outputs/flaps/flaps-serviceable

/controls/engines/engine[0...3]/master-bat

/controls/engines/engine[0...3]/master-alt

/controls/engines/engine[0...3]/magnetos

/controls/engines/engine[0...3]/starter

/controls/engines/engine[0...3]/throttle

/controls/engines/engine[0...3]/mixture

/controls/engines/engine[0...3]/condition

/controls/engines/engine[0...3]/fuel-pump

/controls/engines/engine[0...3]/propeller-pitch

/controls/engines/engine[0...3]/cowl-flaps-norm

/controls/engines/engine[0...3]/ignition

/controls/fuel/tank[0...7]/fuel_selector

/controls/fuel/tank[0...7]/boost-pump[0...1]

/controls/gear/brake-left

/controls/gear/brake-right

/controls/gear/brake-parking

/controls/gear/gear-down

/controls/gear/steering

/controls/lighting/landing-lights

/controls/lighting/taxi-light

/controls/lighting/nav-lights

/controls/lighting/strobe

/controls/lighting/logo-light

/controls/switches/pitot-heat

/sim/freeze/master

/sim/freeze/position

/sim/freeze/fuel

## Modules configuration file

This file is used by the FlightGear Wizard (fgrun).

### Tags description

<rexec>

The <rexec> tag contains the full path and name of the rexec software. Rexec is used to execute distributed applications.

<ModuleList>

The <ModuleList> tag only contains the description of all the modules to launch.

<module>

The <module> tag is used to define a module.

<name>

Name of the module.

<desc>

Description of the module.

<priority>

This tag is used to give an order to launch the module.

Priority of ‘1’ means that the module will be launching first.

If the same priority is given to different modules, the call order is the declaration order.

<host>

This tag gives the name of the computer witch has the module to run.

<bin>

This is the full path with executable name of the module on the host computer.

<kill>

This is the command launched to kill the module on the host computer (full path with executable name).

<param>

This tag is for setting parameters to module

### Defining a module

A module description looks like:

<module>

<run>yes</run>

<name>Sound Module</name>

<desc>Module that plays sound in aircraft cockpit</desc>

<priority>9</priority>

<host>PCUGT30</host>

<bin>"C:/Progra~1/FlightGear/Win32/FlightGear.exe"</bin>

<kill>tskill FlightGear</kill>

<param>

--fg-root="C:\Program Files\FlightGear\data"

--fg-scenery="C:\Program Files\FlightGear\data\Scenery"

--fdm=external

--geometry=620x240

--native-fdm=socket,in,60,,6003,udp

--prop:/sim/rendering/draw-otw=0

--disable-panel

--disable-fgfsrc

</param>

</module>

<run>

- Syntaxe: <run>yes/no</run>

The current module will be launched by supervisor (fgrun) if “yes” is set here.

<name>

Syntaxe: <name>A simple text</name>

This is the name of the module (miscellaneous text).

<desc>

Syntaxe : <desc>A simple text</desc>

This is the description of the module (miscellaneous text).

<priority>

Syntaxe: <priority>1</priority>

This is an arbitrary number for launching the module.

The module with the smallest number will be launch first.

If modules have the same number, they will be launched in the file declaration order.

<host>

Syntaxe: <host>hostname</host>

The module will be launched on the ”hostname” computer.

<bin>

- Syntaxe: <bin>full_path_exe_name</bin>

This tag contains the full path name of the executable file (module).

If the path contains spaces characters it must be in quotation marks (“full path exe name”).

<kill>

Syntaxe: <kill>full_path_cmd_name</kill>

This tag contains the full path name of the command (or executable file) used to kill the module on the host computer. If the path contains spaces characters it must be in quotation marks (“full path cmd name”).

<param>

Syntaxe:

- <param>

parameter1

parameter2

parameter3

…

</param>

These are the arguments used by a module.

For FlightGear modules, these parameters can overwrite the “system.fgfsrc” file parameters.

A new FG option exists “–disable-fgfsrc” to avoid FG from reading the parameters file.

- Special parameter: “?”

- A special parameter is used to retrieve data from FG wizard.

<module>

<run>yes</run>

<name>Sound Module</name>

<desc>Module that plays sound in aircraft cockpit</desc>

<priority>9</priority>

<host>PCUGT30</host>

<bin>"C:/Program Files/FlightGear/bin/Win32/FlightGear.exe"</bin>

<kill>tskill FlightGear</kill>

<param>

--fg-root="C:\Program Files\FlightGear\data"

--fg-scenery="C:\Program Files\FlightGear\data\Scenery"

--fdm=external

--airport=?

--aircraft=?

--runway=?

--timeofday=?

</param>

</module>

In the FG wizard you can select a plane, an airport or modify different properties.

The special parameter “?” applies retrieves data selected in the wizard.

Actually, this parameter only apply on following properties:

- --airport

- --aircraft

- –runway

- --timeofday

In the previous configuration example, airport, aircraft and runway will have the same value you have selected in the wizard interface.

It is very useful when you are running more than one FG visual.

## Force Feedback Configuration files

The Console Acquisition module uses this file.

### FFB main configuration file

This file contains all the aircraft constants and tables needed to compute the three forces to apply on ailerons, elevators and rudders.

Values on this file are relative to the “Airplane Control Force Reproduction in Flight Simulation” document.

For more information on these values, you may have a look to the Force FeedBack chapter for detailed formulas.

#### Amplification

<amplification>

<control name="ELEVATOR">

<gain unit=""> 1.0 </gain>

</control>

<control name="AILERON">

<gain unit=""> 1.0 </gain>

</control>

<control name="RUDDER">

<gain unit=""> 1.0 </gain>

</control>

</amplification>

The amplification group defines a gain value that is applied to the result of the Force Feedback computation.

#### Gearing

<!-- Stick/Yoke position range -->

<positive_position_range_e unit="M"> 0.10 </positive_position_range_e>

<negative_position_range_e unit="M"> 0.20 </negative_position_range_e>

<!-- Yoke Angular range -->

<positive_angular_range_a unit="DEG"> 60.0 </positive_angular_range_a>

<negative_angular_range_a unit="DEG"> 45.0 </negative_angular_range_a>

<!-- Pedal position range -->

<positive_position_range_r unit="M"> 0.16 </positive_position_range_r>

<negative_position_range_r unit="M"> 0.16 </negative_position_range_r>

### FFB configuration tables

This file contains data used by the console acquisition program to pilot motors for the yoke and rudders.

<frictionThreshold>

This value corresponds to the friction of the system (for elevator, ailerons and rudders).

<assistGain>

This gain is applied to the computed force feedback value, to help pilot moving commands.

<softwareBoundLimitationStart>

Limit of the Command position.

<actuatorMaxVoltageSpeed>

This value limits the actuator variation to have a smooth movement.

<actuatorPositiveVoltageDirection>

This value can be “clockwise”, “counterclockwise” for a translation,

“forward”, “backward” for a translation.

This indicates that a positive voltage applied to the actuator match the given direction.

<loadCellPositiveVoltageDirection>

This value can be “clockwise”, “counterclockwise” for a translation,

“forward”, “backward” for a translation.

This indicates that a positive voltage given by the load cell correspond to the direction.

<positionSensorRange>

<minValue> and <maxValue>

These values correspond to the voltage measured when the command is in the min and max position (left position for rotation, the most backward position for translation).

- Conversion tables

This configuration file also contains two mapping tables for each command elevator, ailerons and rudders.

The first table is used to convert the voltage given by each cell to a force value and,

the second one is used to convert a force to a voltage.

WARNING: concerning the aileron, the values given by the tables are the forces applied at the position of the pilot hands on the yoke. These values are not the rotation moment of the yoke.

## ViewCorrection Configuration file

All visual modules use this configuration file.

The view correction is used to compensate the platform movments.

Here is an example of this file:

<?xml version="1.0"?>

<!--

- View correction file

This file is used to configure the 'real' viewing frustums

(the view from pilot and the 3 white projection screens).

- All positions and dimensions are in meters.

Notes:

Hexapod is at origin (0, 0, 0).

The hexapod pivot is at (0, 0, height).

Screen angle is in degree.

-->

<ViewCorr>

<!-- Screen dimension and position -->

<screen_width>4.0</screen_width>

<screen_height>3.0</screen_height>

<screen_bottom>1.45</screen_bottom>

<screen_dist_from_hexapod>1.75</screen_dist_from_hexapod>

<!-- Hexapod pivot position (height) -->

<hexapod_pivot_position>1.5</hexapod_pivot_position>

<!-- Pilot eye position from hexapod pivot -->

<pilot_eye_x>-0.45</pilot_eye_x>

<pilot_eye_y>0.40</pilot_eye_y>

<pilot_eye_z>1.6</pilot_eye_z>

<screen_angle>60.0</screen_angle>

</ViewCorr>

<!-- end of ViewCorrection.xml -->

### Tags description

- All values are in meters.

<screen_width> , <screen_height>

These tags indicate the screen width and height.

<screen_bottom>

This is the distance between the ground and the bottom screen.

<screen_dist_from_hexapod>

This is the distance between the hexapod center and the screen.

<screen_pivot_position>

This tag indicates the position of the pivot from ground.

<pilot_eye_x,y,z>

These tags indicate the position of pilot eye, relative to the pivot.

<screen_angle>

It gives the orientation for left and right screens, from central one.

screen height

screen bottom

distance from

hexapod

pivot position

pilot eye

pivot

## Security Configuration file

The Security module uses this file.

It contains description of the digital inputs (emergency buttons and sensors).

Each channel of the Nationel Instrument acquisition card is affected to a button or a sensor.

This file describes which channel is affected to buttons/sensors.

### Tags description

<delayTime>

The delay time is the time to wait before sending a pause message.

It is use to supervise modules. If a module does not aknowedge during this time, then the simulator is paused.

<channelName>

Channel to use on the National Instrument card.

We actually use the 8 ports of the channel.

<module>

The <module> tag is used to affect a sensor description to a channel of the NI card.

<name>

Button  or sensor name.

<desc>

Description of the button or sensor.

<port>

This tag is used to affect a port to module.

### Defining a module

Here is a configuration example file:

<?xml version="1.0"?>

<!--

- Security Config file

This file is used to associate each security button to a digital input.

************************************************************

The acquisition card is a NI 6220.

Digital I/O port are set between 0 to 7

port 0    is reserved ny security PTF as a output.

port 1-7  are used to freely assign emergency button.

************************************************************

-->

<SecurityList>

<!-- Waiting time before sending emergency stop (in milliseconds) -->

<delayTime>1000.0</delayTime>

<!-- Channel to use on the NI card -->

<channelName>Dev1/port0</channelName>

<!-- ********************** -->

<!-- *   EMERGENCY STOP   * -->

<!-- ********************** -->

<module>

<name>Emergency stop 1</name>

<desc>This button is in the supervisor room</desc>

<port>1</port>

</module>

<module>

<name>Emergency stop 2</name>

<desc>This button is in the supervisor room</desc>

<port>2</port>

</module>

<!-- ******************* -->

<!-- *      DOORS      * -->

<!-- ******************* -->

<module>

<name>Door on room 1</name>

<desc>This contact is on the simulation room</desc>

<port>4</port>

</module>

<module>

<name>Door on room 2</name>

<desc>This contact is on the simulation room</desc>

<port>5</port>

</module>

</SecurityList>

<!-- end of Security.xml -->

## DAQ IO Configuration file

The Console Acquisition module uses this file.

This file is used to configure the National Instruments acquisition Card.

- Here is an example of configuration use for the simulation.

AI	Commands		Dev1/ai0:2			3	-5		5

AI	Commands		Dev1/ai3:5			3	-5		5

AI	Commands		Dev1/ai16:17		2	-5		5

AI	Commands		Dev1/ai18			1	 0		5

AO	ForceFeedBack	Dev1/ao0:2			3	-5		5

DO	Relais		Dev1/port0/line0:3	1

DI	Contact		Dev1/port0/line4		1

- The first column indicates the port type.

| AI | Analogic Input |
| --- | --- |
| AO | Analogic Output |
| DI | Digital Input |
| DO | Digital Output |

- The second column is the name of the channel name.

- The fird column is the physical channel name

Notes:
- ai18  indicates a direct line number (18)
- ai3:5  indicates 3 lines number (3,4 and 5)

- The fourth column indicates the number of lines in the channel

- Then, the two last columns indicate the min and max voltage tension.

# Precision Flight Console

## Controls description

The PFC Cirrus II has the following hardware controls:

| Controls | Description |
| --- | --- |
| Magneto Left | Select the engines on the left aileron (off, right, left, both) |
| Magneto Right | Select the engines on the right aileron (off, right, left, both) |
| Engine Start | Starts the left or right engines |
| Navigation lights | Aircraft lighting (on/off) |
| Strobe lights | Aircraft lighting (on/off) |
| Landing lights | Aircraft lighting (on/off) |
| Taxi lights | Aircraft lighting (on/off) |
| Parking Brake | Set or release parking break (on/off) |
| Battery master | Set the main power to on/off |
| Left alternator | Left alternator on/off |
| Right alternator | Right alternator on/off |
| Avionics master | This button is used with the PFC digital avionics (on/off) |
| Throttle left | Set left throttle (close to open) |
| Throttle right | Set right throttle (close to open) |
| Propeller pitch Left | Increase / decrease left propeller pitch |
| Propeller pitch Right | Increase / decrease right propeller pitch |
| Left Mixture | Set left mixture (rich to lean) |
| Right Mixture | Set right mixture (rich to lean) |
| Flaps | Move flaps up and down |
| Cowl flaps | Move cowl flaps up and down |
| Landing gear | Set landing gear (up or down) |
| Alternate air | Set the alternate air (open, close) |
| Rudder trim | Adjust the rudder trim (left to right) |
| Tank select | Select fuel tank (off, left, both, right) |
| Fuel boost left | Active left fuel boost of selected tank (on / off) |
| Fuel boost right | Active right fuel boost of selected tank (on / off) |
| Pitot heat | Active pitot heat (on / off) |
| Yoke | Moves ailerons and elevators |
| CH controllers on yoke | Moves elevator trim |

- When there is only one engine, only the “left” buttons are used (left throttle, left start…).

## Data send by PFC console

The state of each controls of the PFC console is sent when changes.

Some data is also periodically refreshed for synchronization (each second).

- Not all data is refreshed to avoid serial buffer overflow.

- This means that it’s not possible to have states of few controls when the simulation starts.

Here is data that is not periodically refreshed:

- Magnetos Right engine

- Alternator Left

- Alternator Right

- Avionics master

- Engine start

- Fuel boost right

- Alternate air

- CH controllers

## Controls state when simulation starts

Because all data is not refreshed periodically, the PFC controls must be in a correct state when the simulation is started. The Console Acquisition software initialises data to states described below.

| Controls | Default Position |
| --- | --- |
| Magneto Left | OFF |
| Magneto Right | OFF |
| Engine Start | OFF (default hardware position) |
| Navigation lights | OFF |
| Strobe lights | OFF |
| Landing lights | OFF |
| Taxi lights | OFF |
| Parking Brake | ON |
| Battery master | OFF |
| Left alternator | OFF |
| Right alternator | OFF |
| Avionics master | OFF |
| Throttle left | CLOSE |
| Throttle right | CLOSE |
| Propeller pitch Left | MAX DECREASE |
| Propeller pitch Right | MAX DECREASE |
| Left Mixture | MAX LEAN |
| Right Mixture | MAX LEAN |
| Flaps | DEFAULT hardware position |
| Cowl flaps | DEFAULT hardware position |
| Landing gear | DOWN |
| Alternate air | PUSHED |
| Rudder trim | MIDDLE |
| Tank select | OFF |
| Fuel boost left | OFF |
| Fuel boost right | OFF |
| Pitot heat | OFF |
| Yoke | DEFAULT hardware position |
| CH controllers on yoke | DEFAULT hardware position |

## Console Acquisition Mapping

The table below shows how is mapped PFC controls to flight gear data (default XML file mapping).

| Controls | Description |
| --- | --- |
| Magneto Left | /controls/engines/engine[0]/magnetos /controls/engines/engine[0]/starter (when OFF) |
| Magneto Right | /controls/engines/engine[1]/magnetos /controls/engines/engine[1]/starter (when OFF) |
| Engine Start | /controls/engines/engine[0]/starter (left) /controls/engines/engine[1]/starter (right) |
| Navigation lights | /controls/lighting/nav-lights |
| Strobe lights | /controls/lighting/strobe |
| Landing lights | /controls/lighting/landing-lights |
| Taxi lights | /controls/lighting/taxi-light |
| Parking Brake | /controls/gear/brake-parking |
| Battery master | /controls/engines/engine[0]/master-bat /controls/engines/engine[1]/master-bat |
| Left alternator | /controls/engines/engine[0]/master-al |
| Right alternator | /controls/engines/engine[1]/master-al |
| Avionics master | Not used (PFC controls not send at all) |
| Throttle left | /controls/engines/engine[0]/throttle |
| Throttle right | /controls/engines/engine[1]/throttle |
| Propeller pitch Left | /controls/engines/engine[0]/propeller-pitch |
| Propeller pitch Right | /controls/engines/engine[1]/propeller-pitch |
| Left Mixture | /controls/engines/engine[0]/mixture |
| Right Mixture | /controls/engines/engine[1]/mixture |
| Flaps | /controls/flight/flaps |
| Cowl flaps | /controls/flight/aileron-trim |
| Landing gear | /controls/gear/gear-down |
| Alternate air | Not assigned to FG data |
| Rudder trim | /controls/flight/rudder-trim |
| Tank select | /controls/fuel/tank[0]/fuel_selector /controls/fuel/tank[1]/fuel_selector /controls/engines/engine[0]/starter (when OFF) /controls/engines/engine[1]/starter (when OFF) |
| Fuel boost left | /controls/fuel/tank[selected_tank]/boost-pump[0] |
| Fuel boost right | /controls/fuel/tank[selected_tank]/boost-pump[1] |
| Pitot heat | Not assigned to FG data |
| Yoke | /controls/flight/aileron /controls/flight/elevator |
| CH controllers on yoke | /controls/flight/elevator-trim |

# Console Acquisition parameters

Here is a description of the parameters send to the console acquisition program.

These parameters are set in the Module.xml configuration file in the ConsoleAcquisition section.

COM1

F:/PrecisionFlightControls.xml

F:/FFBconfig.xml

F:/FFBconfigTab.xml

F:/DAQ_IO.cfg

C:/data/input.csv

recordWhenUsingInput=false

--pfc-ctrls=socket,out,120,SUNCTRL1,7000,udp

--pfc-ctrls=socket,out,120,SUNEMBEDED,8010,udp

--ffb-inputs=socket,in,60,,7010,udp

--ffb-outputs=socket,out,120,SUNCTRL1,7080,udp

- Parameter 1: this is the serial communication port used by the Precision Flight Console

- Parameter 2: the second parameter gives the pathname of the Precision Flight Console configuration file

- Parameter 3: this is the first configuration file for the Force Feedback (see Force Feedback configuration file)

- Parameter 4: this is the second configuration file for the Force Feedback (see Force Feedback configuration file)

- Parameter 5: this is the configuration file used to configure the acquisition card (see DAQ IO Configuration file)

- Parameter 6: this parameter is used to give the input file. The input file is used to replay commands recorded in a file. Please refer to chapter 7.

- Parameter 7: this parameter enables or disables the recording of data when using the command for replay. Please refer to chapter 7.

- Parameter 8 and others: these parameters correspond to the Flight Gear Protocols used in the console acquisition

# Record / Replay

## Record / Replay functionality

The console acquisition module gives the possibility to use a “record / replay” functionality.

The record functionality consists in recording specific data into different files. The recording is triggered by a dedicated command on the flight console (the “alternate air” knob). During the recording a flashing red circle is displayed on the dash board.

The replay functionality consists in getting data from a specific input file in order to emulate the actions of the pilot on the aircraft commands according to these data. The replay is triggered by a dedicated command on the flight console (the “pitot heat” knob). During the replay a flashing green triangle is displayed on the dash board and the aircraft commands are not usable by the pilot.

Please note that initial condition of the replay have a significant influence on how the aircraft will react during the replay.

The record produces 3 different files:

- An “initial condition” file that gives the information about the “state” of the aircraft at the moment of the recording.

- An “output” file that gives internal data from the Flight Gear model, the console acquisition and the force feed back calculation. This file is to be used to tune the force feed back module or to analyse the effects of model parameters etc.

- An “input output” file that gives data that can be directly used by the replay function. Hence, each time a record is produced, the manoeuvres of this record can be reproduced by the replay function (and the initial condition file should be used to start the replay with about the same initial condition).

The replay uses an input file that gives time stamped value for the aircraft commands. The values are interpolated according to the time stamps.

Besides, a record can be made during a replay. To do so, the “record when using input” flag should be set in the “Module.xml” console acquisition parameters file (see chapter 6).

When the “record when using input” function is enabled, the record files are written during the replay of an input file.

The purpose of this function is to be able to record data (produced by the Flight Gear model or the force feed back module) with always the same input which simplify the comparison of different sets of data.

## Record output files formats

### “Initial condition” file format

The initial condition file is located in the “C:/data” directory of the embedded PC.

It is named “initial-conditions-YYYY.MM.DD-hh.mm.ss.txt” where:

- YYYY is the year of the record on 4 digits,

- MM is the month of the record on 2 digits,

- DD is the day of the month of the record on 2 digits,

- hh is the hour of the record on 2 digits,

- mm is the minute of the record on 2 digits,

- ss is the second of the record on 2 digits.

The file format is “txt” which means ascii.

The content of the file is:

- air speed=decimal_value kts

- yoke normal translation= decimal_value [-1,+1]

- yoke normal rotation= decimal_value [-1,+1]

- right rudder pedal normal position= decimal_value [-1,+1]

- elevator trim normal position= decimal_value [-1,+1]

- right aileron trim normal position= decimal_value [-1,+1]

- rudder trim normal position= decimal_value [-1,+1]

- left throttle normal position= decimal_value [0,1]

- right throttle normal position= decimal_value [0,1]

- left Mixture normal position= decimal_value [0,1]

- right Mixture normal position= decimal_value [0,1]

- left PropellerPitch normal position= decimal_value [0,1]

- right PropellerPitch normal position= decimal_value [0,1]

- Flaps normal position= decimal_value [0,1]

- GearDown position= integer_value (0 up / 1 down)

- ParkingBrake position integer_value (0 off / 1 on)

- left Brake normal position= decimal_value [0,1]

- right Brake normal position= decimal_value [0,1]

where:

- integer_value is an integer value,

- the possible values of an integer value are given inside “( )”,

- decimal_value is a decimal value,

- the range values of a decimal value are given inside “[ ]”.

### “Output” file format

The output file is located in the “C:/data” directory of the embedded PC.

It is named “output-YYYY.MM.DD-hh.mm.ss.csv” where:

- YYYY is the year of the record on 4 digits,

- MM is the month of the record on 2 digits,

- DD is the day of the month of the record on 2 digits,

- hh is the hour of the record on 2 digits,

- mm is the minute of the record on 2 digits,

- ss is the second of the record on 2 digits.

The file format is “csv” which means an ascii format where the columns of the record are separated by semicolons (“;”).

The first line of the file is the columns title line.

The columns of the file are:

- Time stamp:

  - time (decimal value in seconds),

- Data received from the Flight Gear model:

  - qbar_psf,

  - psidot_rad_sec, thetadot_rad_sec, phidot_rad_sec,

  - model_elevator_pos_rad,model_right_aileron_pos_rad, model_rudder_pos_rad,

  - v_north_fps, v_east_fps, v_down_fps,

  - rdot_rad_sec, qdot_rad_sec, pdot_rad_sec,

  - p_rad_sec, q_rad_sec, r_rad_sec,

  - u_fps, v_fps, w_fps,

  - gz, gy,

  - wdot_fps, vdot_fps,

  - alpha_rad, beta_rad;vt_fps,

  - phi, theta, psi,

  - latitude, longitude, altitude.

- Data produced by the console acquisition and force feed back module:

  - Aileron:

    - RightAileronTrimPosRad, RightAileronTrimValue,

    - Iay, HaInertial, Cha, ChaTrim, HaA,

    - StickPositionA, StickRateA,

    - FaFriction, FFBa,

    - Ga,

    - FFB_RightAileronPosRad (this position is acquired by the console acquisition and updated much quicker than the value extracted from the Fligth Gear model),

    - FaPilot.

  - Elevator:

    - ElevatorTrimPosRad, ElevatorTrimValue,

    - Agz, Gz, Iey, HeInertial, Che, CheTrim, HeA,

    - StickPositionE, StickRateE,

    - FeFriction, FeSpring, FFBe,

    - Ge,

    - FFB_ElevatorPosRad (this position is acquired by the console acquisition and updated much quicker than the value extracted from the Fligth Gear model),

    - FePilot.

  - Rudder:

    - RudderTrimPosRad, RudderTrimValue,

    - Agy, Gy, Irz, HrInertial, Chr, ChrTrim, HrA,

    - PedalPosition, PedalRate,

    - FrFriction, FrSpring, FFBr,

    - Gr,

    - FFB_RudderPosRad (this position is acquired by the console acquisition and updated much quicker than the value extracted from the Fligth Gear model),

    - FrPilot.

  - Throttle:

    - left_throttle, right_throttle,

    - left_mixture, right_mixture,

    - left_PropellerPitch, right_PropellerPitch.

  - Flaps:

    - flaps.

  - Gear:

    - gear_down.

  - Brakes:

    - parking_brake,

    - left_brake, right_brake.

### “Input output” file format

The input output file is located in the “C:/data” directory of the embedded PC.

It is named “input-YYYY.MM.DD-hh.mm.ss.csv” where:

- YYYY is the year of the record on 4 digits,

- MM is the month of the record on 2 digits,

- DD is the day of the month of the record on 2 digits,

- hh is the hour of the record on 2 digits,

- mm is the minute of the record on 2 digits,

- ss is the second of the record on 2 digits.

The file format is the same than the replay input file (see chapter 7.3)

## Replay input file format description

The replay input file must be located on the embedded PC.

The replay input file name is given in the “Module.xml” console acquisition parameters file (see chapter 6). Its default value is “C:/data/input.csv”

The file format is “csv” which means an ascii format where the columns of the record are separated by semicolons (“;”).

There is no columns title line.

The content of each line is:

- Time stamp in seconds (decimal value),

- Right aileron position in degrees (decimal value),

- Right aileron trim position in degrees (decimal value),

- Elevator position in degrees (decimal value),

- Elevator trim position in degrees (decimal value),

- Rudder position in degrees (decimal value),

- Rudder trim position in degrees (decimal value),

- Left throttle lever normalized position (decimal value between 0 and 1),

- Right throttle lever normalized position (decimal value between 0 and 1),

- Left mixture lever normalized position (decimal value between 0 and 1),

- Right mixture lever normalized position (decimal value between 0 and 1),

- Left propeller pitch lever normalized position (decimal value between 0 and 1),

- Right propeller pitch lever normalized position (decimal value between 0 and 1),

- Flaps normalized value (decimal value between 0 and 1),

- Gear down indication (0=up, 1=down),

- Parking brake indication (0=off, 1=on),

- Left brake normalized value (decimal value between 0 and 1),

- Right brake normalized value (decimal value between 0 and 1).

# Force Feedback

## Configuration files

The ConsoleAcquisition is the program that is managing the Force Feedback in the simulation. This program is embedded in the aircraft’s cockpit.

The ConsoleAcquisition program uses two xml configuration files.

The Flight Gear dynamic model does not read these xml files.

- The first one contains all the aircraft constants and tables needed to compute the three forces to apply on ailerons, elevators and rudders.

- The second configuration file contains a two mapping tables.

- The first one is used to have a fast and smooth conversion between Forces and tension (volts) to apply on the control engines.

- The second one is used to have a conversion between tensions given by engine’s sensors to force.

## Coordinate System

### JSBSim (body axes)

Z

Y

X

### FlightGear (aircraft model)

Z

Y

X

## Equations for Elevator

The base equation is the following one:

- FFFB,e = Ge ( H e,inertial + H e,A ) + F e,C

+ F e,friction + F e,spring

| Ge | Gearing ratio |
| --- | --- |
| H e,inertial | Inertial coupling actions |
| H e,A | Aerodynamic hinge moment |
| F e,C | Pilot’s effort, given by the load cell |
| F e,friction | Force due to Friction of the mechanical chain = - σe d( Se )/dt |
| F e,spring | Force due to a spring connected to the yoke = - ke Se |
| Se | Current stick position |
| d( Se )/dt | Current rate of change of stick position |

### Gearing ratio

The gearing ratio can be written as:

- Ge  =  e  /  Se

| e | Elevator position /fdm/jsbsim/fcs/elevator-pos-rad |
| --- | --- |
| Se | Current stick position |

- The stick position is a relative value between -1 and +1.

- The gearing ratio is a constant.

<gearing>

- <control_surface name=”ELEVATOR”>

<Ge unit=”DEG/MT”> 50.0 </Ge>

<!-- Stick position relative to

-half_movement and +half_movement -->

<half_movement unit="M"> 0.2 </half_movement>

<!-- Gearing friction, Damping constant-->

<sigma_e unit=”N*SEC/MT”> 0.001 </sigma_e>

<!-- Spring stiffness -->

<ke unit=”N/MT”> 0.1 </ke>

</control_surface>

</gearing>

### Trim position

The elevator trim angle is defined as the angle between the trim tab and the current (deflected) elevator chord. It is given by:

et = Gec2t e  + Get Set

The result of this computation (et) must be added to the FlightGear variables as:

/oktal/ffb/elevator-trim-pos-rad

/oktal/ffb/elevator-trim-pos-deg

| et | Elevator trim position |
| --- | --- |
| e | Elevator position /fdm/jsbsim/fcs/elevator-pos-rad |
| Gec2t | Elevator “command to trim” gear ratio (servo-tab) |
| Get | Elevator trim gearing ratio |
| Set | Current trim knob position |

The trim knob position is a relative value between -1 and +1.

The gearing ratios are constants.

<gearing>

- <control_surface name=”ELEVATOR”>

<Gec2t unit=””> 0.0 </Gec2t >

- <Get unit=”RAD”> 0.2 </Get>

</control_surface>

</gearing>

### Inertial coupling actions

The inertial coupling actions can be written as:

H e,inertial  =   (q – p r) Iey  –  (aGz – gz) me ee

| q | Pitching acceleration /fdm/jsbsim/accelerations/qdot-rad_sec (please note that this really is an acceleration in rad/s2) |
| --- | --- |
| p | Aircraft body roll rate /fdm/jsbsim/velocities/p-rad_sec |
| r | Aircraft body yaw rate /fdm/jsbsim/velocities/r-rad_sec |
| aGz | Acceleration of Aircraft mass center along standard z-Body axis (opposite to zFG) a_Gz is the component in the standard z-body frame of the the acceleration vector of aircraft mass center (CG) with respect to the earth frame. Being, u, v, w, the CG’s velocity components, uDot, vDot, wDot the corresponding rates, p, q, r the components of the airplane angular velocity vector in the body frame, the acceleration is given by (in ft/s2): aGz = wDot + p v – q u wDot = /fdm/jsbsim/accelerations/wdot-fps (please note that this really is an acceleration in ft/s2) p = /fdm/jsbsim/velocities/p-rad_sec v = /fdm/jsbsim/velocities/v-fps q = /fdm/jsbsim/velocities/q-rad_sec u = /fdm/jsbsim/velocities/u-fps Similarly, the remaining acceleration components are: aGx = uDot + q w – r v ; aGy = vDot + r u – p w |
| gz | Component along the standard z-Body axis of the Vertical Acceleration vector In the file “FGPropagate.cpp”, the earth gravity acceleration is given by : double r = GetRadius(); // radius considering the altitude FGColumnVector3 gAccel( 0.0, 0.0, Inertial->GetGAccel(r) ); // project it in the aircraft body axes : const FGMatrix33& Tl2b = GetTl2b(); // local to body frame transf. matrix // transform to body frame FGColumnVector3 gBodyAccel= Tl2b*gAccel; // get gz gz = gBodyAccel(zAxisIndex); // ft_sec2 |
| me | Elevator Mass <mass> constant </mass> |
| ee | Distance between control surface center of mass and hinge axis <ee> constant </ee> |
| Ie | Moment of inertia around the hinge axis <ie unit=”KG*MT2”> 1.0 </ie> |
| Iey | Product of inertia <iey type=”value”> constant </iey> Or, alternatively <iey type=”calculate”> <lambda_e unit=”DEG”> 20.0 </lambda_e> <xC_e unit=”MT”> -0.07 </Lambda> </iey> and calculated as: Iey = me ee xCe – Ie cos Λe |

<control_mass_balance>

<control_surface name=”ELEVATOR”>

<!-- moment of inertia -->

<ie unit=”KG*M2”> 1.2 </ie>

<!-- products of inertia -->

- <!-- iey unit=”SLUG*FT2”> 106 </iey>

<iey type=”calculate”>

- <lambda_e unit=”DEG”> 10.0 <lambda_e>

<xC_e unit=”MT”> 0.06 <xC_e>

</iey>

<!-- Mass -->

<mass unit=”KG”> 25 </mass>

<!-- Eccentricity -->

<ee unit=”IN”> 1.2 </ee>

</control_surface>

</control_mass_balance>

### Aerodynamic hinge moment

- H e,A  = H q Ce [ Se ( Che + Cheq q^) + Set CheTrim ]

| H | Dynamic press. ratio at Horizontal tail <eta_h> constant </eta_h> |
| --- | --- |
| q | Flight dynamic pressure aero/qbar-psf |
| Se | Area of elevator plan form <ref_area> constant </ref_area> |
| Set | Area of elevator trim plan form <tab_ref_area> constant </tab_ref_area> |
| Ce | Mean chord of portion of elevator and tab <ref_chord> constant </ref_chord> |
| Che | Aerodynamic elevator hinge moment coefficient <2d table> driven by: - angle of attack, - elevator position. |
| CheTrim | Elevator Trim hinge moment coefficient <2d table> driven by: - angle of attack, - elevator trim position. |
| Cheq | Elevator Ch additive component due to q <Cheq> constant </Cheq> |
| q^ | Non-dimensional pitch rate |

With q^ = q Ce  / (2 V )

| q | Aircraft body pitch velocities/q-rad_sec |
| --- | --- |
| Ce | Mean chord of portion of elevator and tab <ref_chord> constant </ref_chord> |
| V | Flight speed /fdm/jsbsim/velocities/vt-fps |

<control_metrics>

<control_surface name=”ELEVATOR”>

- <ref_area  unit=”FT2”> 21 </ref_area>

<tab_ref_area  unit=”MT2”> 0.2 </tab_ref_area>

<ref_chord unit=”FT”> 2.3 </ref_chord>

- <etah> 0.9 </etah>

</control_surface>

</control_metrics>

- Che table

<table>

<independantVar lookup=”row”> /fdm/jsbsim/aero/alpha-rad <independantVar>

<independantVar lookup=”column”> /fdm/jsbsim/fcs/elevator-pos-rad </independantVar>

- <tableData>

...

</tableData>

</table>

- CheTrim table

<table>

<independantVar lookup=”row”> /fdm/jsbsim/aero/alpha-rad    <independantVar>

<!-- Note: trim-pos-rad is the angle between tab and elevator -->

<independantVar lookup=”column”> /oktal/ffb/elevator-trim-pos-rad

</independantVar>

<tableData>

...

</tableData>

</table>

## Equations for Ailerons

The equation looks like elevators one:

FFFB,a = Ga ( H a,inertial + H a,A )  +  F a,C   +  F a,friction

| Ga | Gearing ratio |
| --- | --- |
| H a,inertial | Inertial coupling actions |
| H a,A | Aerodynamic hinge moment |
| F a,C | Pilot’s effort, given by the load cell |
| F a,friction | Force due to Friction of the mechanical chain = - σa d( Sa )/dt |
| Sa | Current lateral stick position |
| d( Sa )/dt | Current rate of change of stick position |

### Aileron Gearing ratio

The gearing ratio can be written as:

- Ga  =  a  /  Sa

| a | Aileron position |
| --- | --- |
| Sa | Current stick lateral position |

The stick position is a relative value between -1 and +1.

The gearing may be thought of as a constant.

<gearing>

- <control_surface name=”AILERONS”>

<Ga unit=”RAD”> 0.01 </Ga>

<-- Stick position relative to

-half_movement and +half_movement -->

<half_movement unit="M"> 0.2 </half_movement>

</control_surface>

</gearing>

### Aileron Trim position

For completeness also the ailerons should be equipped with tabs, as it may happen in some general aviation aircraft.

The aileron trim angle is defined as the angle between the trim tab and the current (deflected) right aileron chord. It is given by:

at = Gac2t a  + Gat Sat

The result of this computation (et) must be added to the FlightGear variables as:

/oktal/ffb/right-aileron-trim-pos-rad

/oktal/ffb/right-aileron-trim-pos-deg

| at | Right aileron trim position |
| --- | --- |
| a | Right aileron position /fdm/jsbsim/fcs/right-aileron-pos-rad |
| Gac2t | Right aileron “command to trim” gear ratio (servo-tab) |
| Gat | Right aileron trim gearing ratio |
| Sat | Right aileron trim knob position |

The trim knob position is a relative value between -1 and +1.

The gearing ratios are constants.

<gearing>

- <control_surface name=”AILERON”>

<Gac2t unit=””> 0.0 </Gac2t >

- <Gat unit=”RAD”> 0.2 </Gat>

</control_surface>

</gearing>

### Inertial coupling actions

The inertial coupling actions can be written as:

H a,inertial  =   - 2 (p + q r) lay

| p | Rolling acceleration /fdm/jsbsim/accelerations/pdot-rad_ sec (please note that this really is an acceleration in rad/s2) |
| --- | --- |
| q | Aircraft body pitch velocities/q-rad_sec |
| r | Aircraft body Heading velocities/r-rad_sec |
| Ia | Aileron total moment of inertia <ia unit=”KG*MT2”> 1.0 </ia> |
| Iay | Aileron total product of inertia <iay type=”value”> constant </iay> Or, alternatively <iay type=”calculate”> <Lambda_a unit=”DEG”> 10.0 </Lambda> <xC_a unit=”MT”> -0.05 </Lambda> </iay> and calculated as: Iay = ma ea xCa – Ia cos Λa |

<control_mass_balance>

<control_surface name=”AILERON”>

<!-- moment of inertia -->

<ia unit=”KG*M2”> 0.9 </ia>

<!-- products of inertia -->

- <iay type=”calculate”>

<lambda_a unit=”DEG”> 15.0 <lambda_a>

- <xC_a unit=”MT”> 0.03 <xC_a>

</iay>

<!-- Mass -->

<mass unit=”KG”> 25 </mass>

<!-- Eccentricity -->

<ea unit=”MT”> 0.02 </ea>

</control_surface>

</control_mass_balance>

### Aerodynamic hinge moment

H a,A  =   q Ca [ Sa Cha + Sat ChaTrim ]

| q | Flight dynamic pressure aero/qbar-psf |
| --- | --- |
| Sa | Area of ailerons plan form <ref_area> constant <ref_area> |
| Sat | Area of aileron trim plan form <tab_ref_area> constant </tab_ref_area> |
| Ca | Mean chord of portion of ailerons and tab <ref_chord> constant <ref_chord> |
| Cha | Aerodynamic ailerons hinge moment coefficient <2d table> driven by : - angle of attack, - right aileron position. |
| ChaTrim | Aileron Trim hinge moment coefficient <2d table> driven by : - angle of attack, - aileron trim position. |

<control_metrics>

<control_surface name=”AILERONS”>

- <ref_area  unit=”FT2”> 13 </ref_area>

<tab_ref_area  unit=”MT2”> 0.2 </tab_ref_area>

<ref_chord unit=”FT”> 0.8 </ref_chord>

</control_surface>

</control_metrics>

- Cha table

<table>

<independantVar lookup=”row”> /fdm/jsbsim/aero/alpha-rad <independantVar>

<independantVar lookup=”column”> /fdm/jsbsim/fcs/right-aileron-pos-rad

</independantVar>

<tableData>

...

</tableData>

</table>

- ChaTrim table

<table>

<independantVar lookup=”row”> /fdm/jsbsim/aero/alpha-rad <independantVar>

<!-- Note: trim-pos-rad is the angle between tab and aileron -->

<independantVar lookup=”column”> /oktal/ffb/right-aileron-trim-pos-rad

</independantVar>

<tableData>

...

</tableData>

</table>

## Equations for Rudders

The equation looks like elevators one:

- FFFB,r = Gr ( H r,inertial + H r,A ) + F r,C

+ F r,friction + F r,spring

| Gr | Gearing ratio |
| --- | --- |
| H r,inertial | Inertial coupling actions |
| H r,A | Aerodynamic hinge moment |
| F r,C | Pilot’s effort, given by the load cell |
| F r,friction | Force due to Friction of the mechanical chain = - σr d( Sr )/dt |
| F r,spring | Force due to a spring connected to the pedals = - kr Sr |
| Sr | Current pedal position |
| d( Sr )/dt | Current rate of change of pedal position |

### Gearing ratio

The gearing ratio can be written as:

- Gr  =  r  /  Sr

| r | Elevator position /fdm/jsbsim/fcs/rudder-pos-rad |
| --- | --- |
| Sr | Current pedal position |

The stick position is a relative value between -1 and +1.

The gearing may be thought of as a constant.

<gearing>

<control_surface name=”RUDDER”>

<Gr unit=”DEG”> 0.003 </Gr>

<!—Pedal position relative to

-half_movement and +half_movement -->

<half_movement unit="M"> 0.2 </half_movement>

<!-- Gearing friction, Damping constant-->

<sigma_r unit=”N*SEC/MT”> 0.001 </sigma_e>

<!-- Spring stiffness -->

<kr unit=”N/MT”> 0.1 </kr>

</control_surface>

</gearing>

### Trim position

The rudder trim angle is defined as the angle between the trim tab and the current (deflected) rudder chord. It is given by:

rt = Grc2t r  + Grt Srt

The result of this computation (rt) must be added to the FlightGear variables as:

/oktal/ffb/rudder-trim-pos-rad

/oktal/ffb/rudder-trim-pos-deg

| rt | Rudder trim position |
| --- | --- |
| r | Rudder position /fdm/jsbsim/fcs/rudder-pos-rad |
| Grc2t | Rudder “command to trim” gear ratio (servo-tab) |
| Grt | Rudder trim gearing ratio |
| Srt | Current trim knob position |

The trim knob position is a relative value between -1 and +1.

The gearing ratios are constants.

<gearing>

<control_surface name=”RUDDER”>

<Grc2t unit=””> 0.0 </Grc2t>

<Grt unit=”RAD”> 0.2 </Grt>

</control_surface>

</gearing>

### Inertial coupling actions

The inertial coupling actions can be written as:

H r,inertial  =    (r + p q ) lrx   +  (r q – p ) lrz  + (aGy – gy) mr er

- Tipically the product of inertia lrx is negligible.

| r | Yaw acceleration /fdm/jsbsim/accelerations/rdot-rad_sec (please note that this really is an acceleration in rad/s2) |
| --- | --- |
| p | Rolling acceleration /fdm/jsbsim/accelerations/pdot-rad_sec (please note that this really is an acceleration in rad/s2) |
| q | Aircraft body pitch rate velocities/q-rad_sec |
| p | Aircraft body roll rate velocities/p-rad_sec |
| r | Aircraft body yaw rate velocities/r-rad_sec |
| aGy | Acceleration of Aircraft mass center along standard y-Body axis See computation for the elevator and finaly get aGy |
| gy | Component along the standard z-Body axis of the Vertical Acceleration vector See computation for the elevator and finaly get gy |
| mr | Rudder Mass <mass> constant </mass> |
| er | Distance between control surface center of mass and hinge axis <er> constant </er> |
| Ir | Moment of inertia around the hinge axis <ir unit=”KG*MT2”> 1.0 </ir> |
| Irz | Product of inertia <irz type=”value”> constant </irz> Or, alternatively <irz type=”calculate”> <lambda_r unit=”DEG”> 20.0 </lambda_r> <xC_r unit=”MT”> -0.07 </xC_r> </irz> and calculated as: Irz = mr er xCr – Ir cos Λr |

<control_mass_balance>

<control_surface name=”ELEVATOR”>

<!-- moment of inertia -->

<ir unit=”KG*M2”> 1.2 </ir>

<!-- products of inertia -->

- <!-- irz unit=”SLUG*FT2”> 106 </irz>

<irz type=”calculate”>

- <lambda_r unit=”DEG”> 10.0 <lambda_r>

<xC_r unit=”MT”> 0.06 <xC_r>

</irz>

<!-- Mass -->

<mass unit=”KG”> 25 </mass>

<!-- Eccentricity -->

<er unit=”IN”> ? </er>

</control_surface>

</control_mass_balance>

### Aerodynamic hinge moment

H r,A  =   Hr q cr [ Sr Chr + (Srt / Sr ) ChrTrim ]

| Hr | Dynamic press. ratio at Vertical tail <eta_hr> constant </eta_hr> |
| --- | --- |
| q | Flight dynamic pressure aero/qbar |
| Sr | Area of elevator plan form <ref_area> constant </ref_area> |
| Srt | Area of rudder trim plan form <tab_ref_area> constant </tab_ref_area> |
| Cr | Mean chord of portion of rudder and tab <ref_chord> constant </ref_chord> |
| Chr | Aerodynamic rudder hinge moment coefficient <2d table> driven by : - angle of attack, - rudder position. |
| ChrTrim | Rudder Trim hinge moment coefficient <2d table> driven by : - angle of attack, - rudder position. |

<control_metrics>

<control_surface name=”RUDDER”>

- <ref_area  unit=”FT2”> 21 </ref_area>

<tab_ref_area  unit=”MT2”> 0.2 </tab_ref_area>

<ref_chord unit=”FT”> 2.3 </ref_chord>

- <etah> 1.0 </etah>

</control_surface>

</control_metrics>

- Chr table

<table>

<independantVar lookup=”row”> /fdm/jsbsim/aero/alpha-rad </independantVar>

<independantVar lookup=”column”> /fdm/jsbsim/fcs/rudder-pos-rad </independantVar>

<tableData>

...

</tableData>

</table>

- Chr trim

<table>

<independantVar lookup=”row”> /fdm/jsbsim/aero/alpha-rad </independantVar>

<!-- Note: trim-pos-rad is the angle between tab and rudder -->

<independantVar lookup=”column”> /oktal/ffb/rudder-trim-pos-rad

</independantVar>

<tableData>

...

</tableData>

</table>

# Compilation

## Module description and libraries dependence

### RExecD

This is a daemon program that is started when Linux / Windows starts.

This program is used to start running and stop execution of the different modules.

Compilation under Windows / Linux.

### TerraGear

This tool is used to manipulate GIS data and convert it into 3d data (models, maps…).

Compilation under Windows / Linux.

Library used :

- SimGear

- Zlib

- Plib

- Gpc

### Scenery Designer

The Scenery Designer is used to modify or create sceneries for Flight Gear.

Compilation under Windows / Linux.

Library used :

- SimGear

- Zlib

- Plib

- Glib

- Gts

- Jpeg

- FLU

- Fltk

### Atlas

Atlas est un programme qui permet l’affichage en vue cartographique des bases de données liées au simulateur Flight Gear.

Compilation under Windows / Linux.

Library used :

- Glut

- lpng

- SimGear

- Zlib

- Plib

- Fltk

### Console Acquisition

This program reads the Precision Flight Console input data, and compute and apply the force feedback to yoke and rudders.

Compilation only under Windows.

Library used :

- FlightGear Lib_Network

- SimGear

- Plib

- NetManager

- NI Acquisition library

### Fgrun (wizard flight gear)

Flight Gear configuration tool and launcher.

Compilation under Windows / Linux.

Library used :

- SimGear

- zlib

- plib

- fltk

- pthreads

### FlightGear

Aircraft Simulator Program.

Compilation under Windows / Linux.

Library used:

- Plib

- SimGear

- AL (Audio Library)

- Zlib

### Security PTF

This program is used to check the Flight Gear model. If the model is not responding before a time delay, the simulation is paused immediately.

Compilation under Windows.

Library used:

- Fltk

- SimGear

- NetManager

- Pthreads

- NI Acquisition library

### Motion Interface

This program is used to pilot the motion platform.

Compilation only under Windows.

Library used:

- CueSim motion library

- FlightGear Lib_Network

- SimGear

- Plib

## Modules / Libraries under Windows

| Modules / Librairies (compilation order) | Visual 2003 Project |
| --- | --- |
| zlib | \zlib\win32\Visual7\zlib.sln |
| plib | \plib\plib.sln |
| simgear | \simgear\SimGear.sln |
| FlightGear | \FlightGear\FlightGear.sln |
| fltk | \fltk\vcnet\fltk.sln |
| pthreads | \pthreads\pthread.sln |
| fgrun | \fgrun\fgrun.sln |
| lpng | \lpng\projects\visualc71\libpng.sln (LIB_debug, LIB_release) |
| Atlas | \Atlas\Visual7\Atlas.sln \Atlas\Visual7\Map.sln |
| TerraGear-0.9.5 | \TerraGear\TerraGear.sln |
| FLU | \FLU\visualc\flulib.sln |
| glib | \glib\build\win32\Visual7\glib.sln |
| gts | \gts\visual7\gts.sln |
| jpeg | \jpeg\libjpeg\libjpeg.sln |
| fgsd-0.2.3 | \fgsd-0.2.3\msvc60\fgsd.sln |
| RExecD | \RexecD\RexecD.sln |
| NetManager | \NetManager\NetManager.sln |
| ConsoleAcquisition | \ConsoleAcquisition\ConsoleAcquisition.sln |
| SecurityPTF | \SecurityPTF\SecurityPTF.sln |
| MotionInterface | \MotionInterface\MotionInterface.sln |

## Modules / Libraries under Linux

To compile all the modules and libraries you need to be the root user and use the csh shell interpreter.

### PLIB

cd plib

setenv CFLAGS    "-O2"

setenv CXXFLAGS  "-O2"

./configure --prefix=/usr/local

make

make install

### ZLIB

cd zlib

make

make install

### SIMGEAR

cd simgear

(aclocal)

(autoheader)

(automake)

(autoconf)

setenv CFLAGS    "-O2"

setenv CXXFLAGS  "-O2"

./configure --prefix=/usr/local

make

make install

### FLIGHTGEAR

cd FlightGear

(aclocal)

(autoheader)

(automake)

(autoconf)

setenv CFLAGS    "-O2"

setenv CXXFLAGS  "-O2"

./configure --prefix=/usr/local

make

make install

### FLTK

cd fltk

setenv CFLAGS    "-O2 -DHAVE_PTHREAD"

setenv CXXFLAGS  "-O2 –DHAVE_PTHREAD"

./configure --prefix=/usr/local

make

make install

### FGRUN

cd fgrun

(autogen.sh)

(aclocal)

(autoheader)

(automake)

(autoconf)

setenv CFLAGS    "-O2"

setenv CXXFLAGS  "-O2"

./configure --prefix=/usr/local

make

make install

### ATLAS

cd Atlas

(aclocal)

(autoheader)

(automake)

(autoconf)

./configure --prefix=/usr/local

make

make install

### FLU

cd FLU

./configure --prefix=/usr/local

make

make install

### GLIB

cd glib

./configure --prefix=/usr/local

make

make install

### LPNG

cd lpng

./configure --prefix=/usr/local

make

make install

### GTS

setenv LIBDIR             /usr/local/lib

setenv LD_LIBRARY_PATH    $LIBDIR

cd gts

rm src/config.h

./configure

make

make install

### JPEG

cd jpeg

make

make install

### NURBS++

cd nurbs++

make

make install

### TerraGear

cd TerraGear-0.9.5

./configure --prefix=/usr/local

make

make install

### fgsd (Scenery DESIGNER)

cd fgsd-0.2.3

./configure --prefix=/usr/local

make

make install

# Source code modifications

All source code that was modified by OKTAL is surrounded by comments like this:

// OKTAL Specific Begin

new source code...

// OKTAL Specific End

## FlightGear

We have modified FlightGear source code to integrate it into the simulation, to support the motion View Correction, and then to have new protocols and options…

### New protocols

| Protocol names | Description |
| --- | --- |
| ffb-inputs | This protocol is used to send some FlightGear / JSBSim data to the Console Acquisition program. This is used in the computation of the force feedback. |
| ffb-outputs | This protocol is used to send to the FlightGear all internal variables used in the computation of the force feedback, such as pilot force, inertial hinge moment… |
| pfc-ctrls | This is the protocol used to reflect the Precision Flight Console use controls. It also contains yoke and rudders position. |
| view-correction | This protocol is used between the motion interface program and the 3 visuals. This is the view correction data to apply on visuals. |

### New options

| Options names | Description |
| --- | --- |
| disable-fgfsrc | This option disables the read of the FlightGear configuration file if it exists. |
| view-correction-cfg-file | This option gives the configuration file to use for the view correction. |
| wizard-server | This option gives the computer hostname and port id of the wizard server (fgrun). |
| security-server | This option gives the computer hostname and port id of the security server (SecurityPTF). |
| module-name | This gives a name that is displayed in the security platform. |

### Shared memory

A new medium was added to Flight Gear, the Shared Memory (shm).

It allows Flight Gear to transfer data such as socket or serial, but it is limited to processes on the same computer.

- Currently, the shm is not used in the simulation.

### “Watch dog” support

The purpose of the watchdog is to regularly check the life of an application.

In this case, we need to check the Flight Gear model, which is a very important component of the simulation.

The watchdog posts a semaphore and wait for the checked application to release it.

If the semaphore is not released after one second, an emergency stop occurred.

- Currently, the “watch dog” is not used in the simulation.

- The security PTF is used to perform this task throw network (TCP/IP sockets).

### Change to the Windows Visual project

We have modified the Visual 2003 FlightGear solution to externalize all protocols in a lib.

This new lib is used on different program such as the console acquisition.

## Fgrun

The fgrun program was also modified. Wa added a new page, to start, stop or pause the simulation.

## SimGear

We have modified clouds generation in SimGear to make clouds continuous on the three visuals.

## Atlas

We have modified Atlas source code to integrate it into the simulation, and then to be able to move the window on the secondary tactile screen.

- This new argument option was added:  --position=1024,0

# Annexes

## JSBSim properties

Here are some usefull property names.

=======================================================

Here are the JSBSim INPUT (FROM flightgear) properties:

- Flight Control

--------------

gear/gear-cmd-norm

fcs/aileron-cmd-norm

fcs/elevator-cmd-norm

fcs/rudder-cmd-norm

fcs/spoiler-cmd-norm

fcs/speedbrake-cmd-norm

fcs/flap-cmd-norm

fcs/pitch-trim-cmd-norm

fcs/yaw-trim-cmd-norm

fcs/roll-trim-cmd-norm

Autopilot (Highly Experimental, "JSBSim only" I suspect)

-------------------------------

ap/acquire_airspeed

ap/acquire_altitude

ap/acquire_attitude

ap/acquire_heading

ap/aileron_cmd

ap/airspeed_hold

ap/airspeed_setpoint

ap/altitude_hold

ap/altitude_setpoint

ap/attitude_hold

ap/attitude_setpoint

ap/elevator_cmd

ap/heading_hold

ap/heading_setpoint

ap/rudder_cmd

ap/throttle_cmd

ap/wingslevel_hold

Here are the JSBSim OUTPUT (Internal use, or TO flightgear) properties:

- Flight Control

--------------

fcs/elevator-pos-norm

fcs/elevator-pos-rad

fcs/flap-pos-deg

fcs/flap-pos-norm

fcs/left-aileron-pos-norm

fcs/left-aileron-pos-rad

fcs/mag-elevator-pos-rad

fcs/mag-left-aileron-pos-rad

fcs/mag-right-aileron-pos-rad

fcs/mag-rudder-pos-rad

fcs/mag-speedbrake-pos-rad

fcs/mag-spoiler-pos-rad

fcs/right-aileron-pos-norm

fcs/right-aileron-pos-rad

fcs/rudder-pos-norm

fcs/rudder-pos-rad

fcs/speedbrake-pos-norm

fcs/speedbrake-pos-rad

fcs/spoiler-pos-norm

fcs/spoiler-pos-rad

gear/gear-pos-norm

gear/num-units

propulsion/active_engine

propulsion/cutoff_cmd

propulsion/magneto_cmd

propulsion/starter_cmd

- Dynamics

--------

aero/alpha-deg

aero/alpha-max-deg

aero/alpha-min-deg

aero/alpha-rad

aero/alpha-wing-rad

aero/alphadot-deg_sec

aero/alphadot-rad_sec

aero/beta-deg

aero/beta-rad

aero/betadot-deg_sec

aero/betadot-rad_sec

aero/bi2vel

aero/ci2vel

aero/cl-squared-norm

aero/h_b-cg-ft

aero/h_b-mac-ft

aero/mag-beta-deg

aero/mag-beta-rad

aero/qbar-psf

aero/qbarUV-psf

aero/qbarUW-psf

aero/stall-hyst-norm

accelerations/a-pilot-x-ft_sec2

accelerations/a-pilot-y-ft_sec2

accelerations/a-pilot-z-ft_sec2

accelerations/n-pilot-x-norm

accelerations/n-pilot-y-norm

accelerations/n-pilot-z-norm

accelerations/pdot-rad_sec

accelerations/qdot-rad_sec

accelerations/rdot-rad_sec

accelerations/udot-fps

accelerations/vdot-fps

accelerations/wdot-fps

attitude/heading-true-rad

attitude/phi-rad

attitude/pitch-rad

attitude/psi-rad

attitude/roll-rad

attitude/theta-rad

flight-path/gamma-rad

flight-path/psi-gt-rad

position/epa-rad

position/h-agl-ft

position/h-sl-ft

position/lat-gc-rad

position/long-gc-rad

position/radius-to-vehicle-ft

velocities/h-dot-fps

velocities/mach-norm

velocities/machU

velocities/p-aero-rad_sec

velocities/p-rad_sec

velocities/phidot-rad_sec

velocities/psidot-rad_sec

velocities/pt-lbs_sqft

velocities/q-aero-rad_sec

velocities/q-rad_sec

velocities/r-aero-rad_sec

velocities/r-rad_sec

velocities/tat-c

velocities/tat-r

velocities/thetadot-rad_sec

velocities/u-aero-fps

velocities/u-fps

velocities/v-aero-fps

velocities/v-down-fps

velocities/v-east-fps

velocities/v-fps

velocities/v-north-fps

velocities/vc-fps

velocities/vc-kts

velocities/ve-fps

velocities/ve-kts

velocities/vg-fps

velocities/vt-fps

velocities/w-aero-fps

velocities/w-fps

lat-gc-deg

lat-gc-rad

long-gc-deg

long-gc-rad

radius-ft

- Environment

-----------

atmosphere/P-psf

atmosphere/P-sl-psf

atmosphere/T-R

atmosphere/T-sl-R

atmosphere/a-fps

atmosphere/a-norm

atmosphere/a-sl-fps

atmosphere/crosswind-fps

atmosphere/delta-norm

atmosphere/headwind-fps

atmosphere/p-turb-rad_sec

atmosphere/psiw-rad

atmosphere/q-turb-rad_sec

atmosphere/r-turb-rad_sec

atmosphere/rho-sl-slugs_ft3

atmosphere/rho-slugs_ft3

atmosphere/sigma-norm

atmosphere/theta-norm

- Forces and Moments

------------------

forces/fbx-aero-lbs

forces/fbx-gear-lbs

forces/fbx-prop-lbs

forces/fbx-total-lbs

forces/fby-aero-lbs

forces/fby-gear-lbs

forces/fby-prop-lbs

forces/fby-total-lbs

forces/fbz-aero-lbs

forces/fbz-gear-lbs

forces/fbz-prop-lbs

forces/fbz-total-lbs

forces/fwx-aero-lbs

forces/fwy-aero-lbs

forces/fwz-aero-lbs

forces/lod-norm

moments/l-aero-lbsft

moments/l-gear-lbsft

moments/l-prop-lbsft

moments/l-total-lbsft

moments/m-aero-lbsft

moments/m-gear-lbsft

moments/m-prop-lbsft

moments/m-total-lbsft

moments/n-aero-lbsft

moments/n-gear-lbsft

moments/n-prop-lbsft

moments/n-total-lbsft

- Initialization

--------------

ic/alpha-deg

ic/alpha-rad

ic/beta-deg

ic/beta-rad

ic/gamma-deg

ic/gamma-rad

ic/h-agl-ft

ic/h-sl-ft

ic/lat-gc-deg

ic/lat-gc-rad

ic/long-gc-deg

ic/long-gc-rad

ic/mach-norm

ic/p-rad_sec

ic/phi-deg

ic/phi-rad

ic/psi-true-deg

ic/psi-true-rad

ic/q-rad_sec

ic/r-rad_sec

ic/roc-fpm

ic/roc-fps

ic/sea-level-radius-ft

ic/terrain-altitude-ft

ic/theta-deg

ic/theta-rad

ic/u-fps

ic/v-fps

ic/vc-kts

ic/ve-kts

ic/vg-fps

ic/vg-kts

ic/vt-fps

ic/vt-kts

ic/vw-bx-fps

ic/vw-by-fps

ic/vw-bz-fps

ic/vw-dir-deg

ic/vw-down-fps

ic/vw-east-fps

ic/vw-mag-fps

ic/vw-north-fps

ic/w-fps

- Aircraft Properties

-------------------

inertia/cg-x-ft

inertia/cg-y-ft

inertia/cg-z-ft

inertia/mass-slugs

inertia/weight-lbs

metrics/Sh-sqft

metrics/Sv-sqft

metrics/Sw-sqft

metrics/aero-rp-x-in

metrics/aero-rp-y-in

metrics/aero-rp-z-in

metrics/bw-ft

metrics/cbarw-ft

metrics/eyepoint-x-in

metrics/eyepoint-y-in

metrics/eyepoint-z-in

metrics/iw-deg

metrics/lh-ft

metrics/lh-norm

metrics/lv-ft

metrics/lv-norm

metrics/runway-radius

metrics/vbarh-norm

metrics/vbarv-norm

metrics/visualrefpoint-x-in

metrics/visualrefpoint-y-in

metrics/visualrefpoint-z-in

systems/stall-warn-norm

value-lbs

## Flight Console Protocol

- Precision Flight Controls Inc. gave this information for an internal use only.

The University of Naples and OKTAL have signed a Non Disclosure Agreement about it.

- Precision Flight Controls

Generic Serial Protocol Definition

Rev 1.7 2/25/2004

0. REVISION HISTORY

1.0	11/4/97		Initial version

1.0A	11/13/97	Added Transponder Ident button codes

1.0B	12/15/97	Added protocol switching

1.0C	3/14/98		Added GPS/Alt Preselect control inputs

and autopilot test button;

Added Pro Panel information;

Added more Control State and Digital

Control inputs;

Modified Cowl flaps Control State message

1.0D	3/15/98		Clarification of Pro vs Avionics panels

Removed unsupported decimal point option

after first display digit

1.0E	6/19/98		Changed definitions of magneto positions

1.0F	6/25/98		Added more ADF and DME panel switch inputs

1.0G	9/14/98		Changed DME switch input descriptions to

match hardware change to slide

switches instead of pushbuttons

Added "fast mode" for console inputs

1.0H	4/9/99		Added landing gear output (table 10)

1.0J	6/9/99		Added info on ADF indicators (table 9)

1.0K	8/1/99		Added GPS ALT/VS preselect toggle input

1.0L	10/2/99		Added info on DME indicators (table 9)

1.0M	11/6/99		Added info on Transponder Ident light

(table 9)

1.0N	12/2/99		Revised DME indicators (table 9)

1.0P	2/18/00		Revised ADF indicators (table 9)

1.0Q	4/1/00		Added AP cmd with AP light off (table 7)

1.0R	6/10/00		Added CH controller coding (table 2)

1.1	9/28/00		Corrected document errors in table 7

1.2	6/23/01		Added Jetliner controls (table 2)

1.3	9/13/02		Added Electric pitch trim output (table 11)

USB notes (section 1)

1.4	6/18/03		Added analog elevator trim (table 1)

1.5	1/9/04		Additions for custom console

1.6	1/30/04		Additions for jet cockpit

1.7	2/25/04		Updates for jet cockpit

- 1. GENERAL

- This document defines the generic native serial protocol supported

by the Precision Flight Controls (PFC) Cirrus II control panel, Avionics

panel and Jetliner control panel. Any of these devices can be used

on their own, or the Avionics panel can be piggy-backed into either

control panel.

The control panels accept user inputs from several sources

and generate serial messages to the host computer/flight simulator

indicating changes to those inputs.

- The Avionics panel also generates serial messages in response to

user inputs. In addition, the Avionics panel accepts display change

serial commands from the host computer/flight simulator and maintains

corresponding displays (digits and status lights).

- The serial interface is a standard three-wire RS232 DTE port on

a DB9 female connector:

pin 5 = ground

pin 2 = transmit data (panel to host)

pin 3 = receive data (host to panel)

(This connects to a 9-pin PC serial port using a straight-through

cable.) The interface is 9600 baud, 8 data bits, no parity, 1 stop

bit. There is no hardware or software flow control, nor any

error detection coding. The panels handle debouncing of switch

hardware, so that they only send one message per valid input change.

- Messages are also sent periodically to indicate the current state

of non-momentary controls (e.g. toggle switches).

- All panels support multiple serial protocols. The control panels

have a physical switch that must be set correctly to enable the

native protocol described in this document. The Avionics panel

can be jumpered as well, or it can automatically switch protocol

in software. If not jumpered to native protocol, the Avionics panel

defaults to a different protocol than the one described here -

to activate the native protocol, the host computer MUST send the Select

- Protocol message defined in section 3. Any data received from

the panel before this message has been sent should be ignored.

- If the panel is equipped with a USB interface, a standard USB

cable replaces the serial cable. A driver on the PC creates a

virtual serial port and translates USB panel messages to appear

as if the panel is connected on that serial port, so the PC

application can use the same programming interface as before,

other than perhaps using an unusual port such as COM3 or COM4.

- 2. PANEL-TO-HOST MESSAGES

- Messages sent by the panel to the host corresponding to user input

changes consist of one or two bytes. The first byte always has a

most-significant bit set to one; if there is a second byte, it will

always have a most-significant bit of zero. This allows the host

to synchronize with the data stream when it first gets connected.

- The high-order 4 bits of the first byte indicate the general source

for the message, and whether or not a second byte will follow. The

low-order 4 bits of the first byte indicate the specific source.

The messages in hexadecimal are:

8X = analog control input X, followed by value (see Table 1)

9X = control state change X, followed by value (see Table 2)

AX = simulator control button X released (see Table 3)

BX = simulator control button X pressed (see Table 3)

CX = digital control input X released (see Table 4)

DX = digital control input X pressed (see Table 4)

EX = panel device X "up", followed by switch ID (see Table 5)

FX = panel device X "down", followed by switch ID (see Table 5)

-------------------------------------------------------------

- Table 1: Analog Control Inputs

- These are two-byte messages, where the first byte is 80 hex +

channel ID, and the second byte is the analog value 00-7F hex.

They are sent when the control inputs change, and are also

sent periodically (every few seconds) for synchronization.

The actual range of analog inputs depends on the control device

hardware - the host should perform a calibration process on

each input and scale the values according to the device limits

(the panel may also provide an analog trim adjustment).

Trim data may also be sent as a separate channel, depending on

the device.

first byte		value byte

80		Ailerons	00 -> 7F = left -> right

81		Elevators	00 -> 7F = down -> up

82		Rudder		00 -> 7F = left -> right

83		Mixture left	00 -> 7F = rich -> lean

84		Prop/RPM left	00 -> 7F = high -> low

85		Throttle left	00 -> 7F = full -> idle

86		Brake left	00 -> 7F = pressed -> released

87		Mixture right	00 -> 7F = rich -> lean

88		Prop/RPM right	00 -> 7F = high -> low

89		Throttle right	00 -> 7F = full -> idle

8A		Brake right	00 -> 7F = pressed -> released

8B		Elevator trim	00 -> 7F = down -> up

8C		Aileron trim	00 -> 7F = left -> right

8D		Rudder trim	00 -> 7F = left -> right

8E		Steering tiller 00 -> 7F = left -> right

Examples:

80 40	= ailerons centered

85 60	= left throttle about 25%

-------------------------------------------------------------

- Table 2: Control State Changes

- These are two-byte messages, where the first byte is 90 hex +

channel ID, and the second byte indicates the new state.

They are sent when the control inputs change, and are also

sent periodically (every few seconds) for synchronization.

Some devices that are exclusive to either prop or jet panels

use the same first byte, but have different second byte

values in case the host software wants to distinguish between them.

first byte			value byte

90		Flaps		00=hold, 01=raise, 02=lower

91 (prop)	Cowl flaps L	00=hold, 01=close, 02=open

91 (jet)	APU		10=off, 11=on, 12=start

92 (prop)	Engine start	00=inactive, 01=left, 02=right

92 (jet)	Eng start 1/2	10 +

bit 0-1=engine 1: 00=off, 01=on, 10=start

bit 2-3=engine 2: 00=off, 01=on, 10=start

92 (jet)	Eng start 3/4	20 +

bit 0-1=engine 3: 00=off, 01=on, 10=start

bit 2-3=engine 4: 00=off, 01=on, 10=start

92 (jet B)	Engine 1 	30 +

0=off, 1=flt, 2=grd, 3=cont

92 (jet B)	Engine 2	40 +

0=off, 1=flt, 2=grd, 3=cont

93		Fuel select L	00=off, 01=left, 02=L+R, 03=right

93 (jet)	Ctr Fuel Pump L 20=off, 21=on

93 (jet)	#1 Fuel Pump Fwd 30=off, 31=on

93 (jet)     #2 Fuel Pump Fwd 40=off, 41=on

94		Transponder mode 00=off, 01=standby, 02=on, 03=alt,

04=test

95		Marker Beacon	00=off, 01=low, 02=high

96 (prop)	Cowl flaps R	00=hold, 01=close, 02=open

96 (jet)	Auto brake	10 + position 0-5

21/20 = auto brake ENG pressed/released

41/40 = auto brake SYS pressed/released

97		Fuel select R	00=off, 01=left, 02=L+R, 03=right

97		Fuel X-feed	10=off, 11=left, 12=right

97 (jet)	Ctr Fuel Pump R 20=off, 21=on

97 (jet)	#1 Fuel Pump Aft 30=off, 31=on

97 (jet)     #2 Fuel Pump Aft 40=off, 41=on

98		Magneto L eng	00=off, 01=Rmag, 02=Lmag, 03=both

99		Magneto R eng	00=off, 01=Rmag, 02=Lmag, 03=both

9A		Lights A	bit 0 (LSB) = collision (1=on)

bit 1 = navigation (1=on)

bit 2 = landing (1=on)

bit 3 = taxi (1=on)

bit 4 = logo (1=on)

bit 5 = wing (1=on)

9A		Lights B	40 +

bit 0 = landing inboard left (1=on)

bit 1 = landing inboard right (1=on)

bit 2 = landing outboard left (1=on)

bit 3 = landing outboard left (1=on)

bit 4 = runway turnoff left (1=on)

bit 5 = runway turnoff right (1=on)

9B		CH controller	bit 0-3 = switch code

9C		Jet switches	bit 0 (LSB) = auto throttle (1=on)

bit 1 = (wing) de-ice (1=on)

bit 2 = EICAS (1=on)

bit 3 = go around (1=on)

bit 4 = engine 1 anti-ice (1=on)

bit 5 = wing anti-ice test (1=test)

bit 6 = engine 2 anti-ice (1=on)

9D		Lights C	bit 0-1 = strobe

(00=off, 01=on, 10=steady)

bit 2 = wheel well (1=on)

bit 3 = no smoking (1=on)

bit 4 = fasten seat belts (1=on)

9F		Custom toggles	00 = slew off, 01 = slew on

10 = prop syn off, 11 = prop syn on

20 = view, 21=normal, 22=cycle

30 = pause off, 31 = pause on

The CH controller bits 0-3 correspond to DB15 game port button inputs

A1, A2, B1, and B2 respectively. A typical assignment is:

00 = view up

01 = view down

02 = view right

03 = view left

05 = button X1

06 = button X2

07 = button A1

08 = trim up

09 = trim down

0A = trim right

0B = button A2

0C = trim left

0D = button B1

0E = button B2

0F = none

Examples:

90 01	= raise flaps

90 00	= stop raising flaps

93 03	= fuel select both left and right

9A 06	= navigation and landing lights on, other lights off

-------------------------------------------------------------

- Table 3: Simulator Button Changes

- These are one-byte messages, used for simulator

control functions like pause/resume or screen selections.

- Code BX is sent when pushbutton X on the main control panel

is pushed, and code AX is sent when button X is released.

- (It might be appropriate to change the labels on these

buttons and the corresponding message meaning.) Most likely,

the simulator can ignore all the "released" (AX) codes.

- These functions are supported by the Avionics Panel but not

the control panels.

Code		Meaning

B0/A0	Clock reset pressed/released

B1/A1	Clock start pressed/released

B2/A2	Instrument screen pressed/released

B3/A3	Map screen pressed/released

B4/A4	Control screen/Menu pressed/released

B5/A5	Freeze (pause/resume) pushbutton pressed/released

B6/A6	Replay pressed/released

B7/A7	Sound pressed/released

B8/A8	Zoom Reset pressed/released

B9/A9	Snapshot pressed/released

BA/AA	Aero Vectors pressed/released

BF/AF	Exit Sim pressed/released

Examples:

B5 A5	freeze button pushed once (pause simulation)

B5 A5	freeze button pushed again (resume simulation)

-------------------------------------------------------------

- Table 4: Digital Control Input Changes

These are one-byte messages corresponding to discrete control

input changes. Code DX is sent when input X is pressed/activated,

and code CX is sent when input X is released. The current state

is also resent periodically (every few seconds) for synchronization.

Code		Meaning

D0/C0	Pitch Trim Down pressed/released

D1/C1	Pitch Trim Up pressed/released

C2	Lower Landing Gear

D2	Raise Landing gear

D3/C3	Radio on/off

D4/C4	Carb Heater on/off

D5/C5	Parking Brake on/off

D6/C6	Fuel Boost left on/off

D7/C7	Pitot Heater on/off (see also FB43/44)

D8/C8	CWS pressed/released

D9/C9	Fuel Boost right on/off

DA/CA	Alternate air on/off

DB/CB	Battery master on/off

DC/CC	Radio master on/off

DD/CD	Alternator L on/off

DE/CE	Alternator R on/off

DF/CF	Autopilot toggle on/off

Examples:

C2	= lower landing gear

C6	= turn off fuel boost

-------------------------------------------------------------

- Table 5: Panel Switch Inputs

- These are two-byte messages indicating a pushbutton or rotary encoder

change on one of the panel devices. When a button is pushed

on device X, or an encoder is turned in a counterclockwise

direction, code FX ("down") is sent, followed by a byte indicating

the specific switch or encoder on device X. When the button is

released, or an encoder is turned in a clockwise direction,

code EX ("up") is sent, followed by the switch/encoder ID.

- While the panel supports two remote consoles, inputs on either

one generate the same messages. Console encoder inputs generate

- "slow" messages when turned slowly, and "fast" messages when

turned quickly. Ideally, the slow messages should result in

fine increments, and the fast messages result in coarse increments

- (about 10 times the fine increments). If this is a problem,

both slow and fast messages can be treated the same way.

- Device IDs

0	COM1

1	NAV1

2	COM2

3	NAV2

4	DME

5	ADF

6	Autopilot

7	Transponder

8	Remote console

9	GPS/Altitude preselect

A	Custom panel

B	Jet console

First byte	Second byte

F0/E0		00 = COM1 On/Off pressed/released

F0/E0		01 = COM1 Active/Standby pressed/released

F0/E0		02 = COM1 Speaker pressed/released

F0/E0		03 = COM1 Coarse Frequency select down/up

F0/E0		04 = COM1 Fine Frequency select down/up

F1/E1		00 = NAV1 Radial pressed/released

F1/E1		01 = NAV1 Active/Standby pressed/released

F1/E1		02 = NAV1 Ident pressed/released

F1/E1		03 = NAV1 Coarse Frequency select down/up

F1/E1		04 = NAV1 Fine Frequency select down/up

F2/E2		00 = COM2 On/Off pressed/released

F2/E2		01 = COM2 Active/Standby pressed/released

F2/E2		02 = COM2 Speaker pressed/released

F2/E2		03 = COM2 Coarse Frequency select down/up

F2/E2		04 = COM2 Fine Frequency select down/up

F3/E3		00 = NAV2 Radial pressed/released

F3/E3		01 = NAV2 Active/Standby pressed/released

F3/E3		02 = NAV2 Ident pressed/released

F3/E3		03 = NAV2 Coarse Frequency select down/up

F3/E3		04 = NAV2 Fine Frequency select down/up

F4		00 = DME mode1 set to OFF

F4		05 = DME mode1 set to R1 (Remote 1)

F4		06 = DME mode1 set to R2 (Remote 2)

F4		01 = DME mode2 set to Ground Speed/Time

F4		07 = DME mode2 set to RMT (remote)

F4		08 = DME mode2 set to Frequency

F4/E4		02 = DME Ident pressed/released

F4/E4		03 = DME Coarse Frequency select down/up

F4/E4		04 = DME Fine Frequency select down/up

F4		09 = DME RMI = NAV2 receiver

E4		09 = DME RMI = NAV1 receiver

F5/E5		00 = ADF On/Off pressed/released

F5/E5		01 = ADF Active/Standby pressed/released

F5/E5		02 = ADF Ident pressed/released

F5/E5		03 = ADF Coarse Frequency select down/up

F5/E5		04 = ADF Fine Frequency select down/up

F5/E5		05 = ADF ADF (antenna mode) pressed/released

F5/E5		06 = ADF BFO pressed/released

F5/E5		07 = ADF FLT pressed/released

F5/E5		08 = ADF SET pressed/released

F6/E6		00 = Autopilot On/Off pressed/released

F6/E6		01 = Autopilot Trim Down pressed/released

F6/E6		02 = Autopilot Trim Up pressed/released

F6/E6		03 = Autopilot FD  pressed/released

F6/E6		04 = Autopilot ALT pressed/released

F6/E6		05 = Autopilot HDG pressed/released

F6/E6		06 = Autopilot NAV pressed/released

F6/E6		07 = Autopilot APR (APP) pressed/released

F6/E6		08 = Autopilot BC  pressed/released

F6/E6		09 = Autopilot Test pressed/released

F6/E6		0A = Autopilot Autothrottle pressed

F7		00 = Transponder mode set to OFF*

F7		01 = Transponder mode set to STANDBY*

F7		02 = Transponder mode set to ON*

F7		03 = Transponder mode set to ALT*

F7		04 = Transponder mode set to TEST*

F7/E7		05 = Transponder digit 1 down/up

F7/E7		06 = Transponder digit 2 down/up

F7/E7		07 = Transponder digit 3 down/up

F7/E7		08 = Transponder digit 4 down/up

F7/E7		09 = Transponder Ident pressed/released

F8/E8		00 = Console ADF course select down/up (slow)

F8/E8		10 = Console ADF course select down/up (fast)

F8/E8		01 = Console HSI course select down/up (slow)

F8/E8		11 = Console HSI course select down/up (fast)

F8/E8		02 = Console heading select down/up (slow)

F8/E8		12 = Console heading select down/up (fast)

F8/E8		03 = Console ALT/QNH select down/up (slow)

F8/E8		13 = Console ALT/QNH select down/up (fast)

F8/E8		04 = Console OBS1 bearing down/up (slow)

F8/E8		14 = Console OBS1 bearing down/up (fast)

F8/E8		05 = Console OBS2 bearing down/up (slow)

F8/E8		15 = Console OBS2 bearing down/up (fast)

F9/E9		00 = GPS ON down/up

F9/E9		01 = GPS NAV/GPS down/up

F9/E9		02 = GPS NAV down/up

F9/E9		03 = GPS WPT down/up

F9/E9		04 = GPS FPL down/up

F9/E9		05 = GPS CALC down/up

F9/E9		06 = GPS AUX down/up

F9/E9		07 = GPS Dir To down/up

F9/E9		08 = GPS MSG down/up

F9/E9		09 = GPS ENT down/up

F9/E9		0A = GPS APT/VOR down/up

F9/E9		0B = GPS coarse select down/up

F9/E9		0C = GPS fine select down/up

F9/E9		0D = Alt Preselect ENG VS down/up

F9/E9		0E = Alt Preselect ALT ARM down/up

F9/E9		0F = Alt Preselect SET (Alt mode) down/up

F9/E9		10 = Alt Preselect SET (V/S mode) down/up

F9		11 = Alt mode toggle

E9		11 = V/S mode toggle

EA		00 = Increments released

FA		00 = Increments +30min pressed

FA		01 = Increments -30min pressed

EA		02 = Slew ALT/ALL released

FA		02 = Slew ALT pressed

FA		03 = Slew ALL pressed

EA		04 = Slew CW/CCW released

FA		04 = Slew CCW pressed

FA		05 = Slew CW pressed

EA		06 = Slew Freeze released

FA		06 = Slew Freeze ALL pressed

FA		07 = Slew Freeze ALT pressed

EA		08 = Slew up/down released

FA		08 = Slew Up pressed

FA		09 = Slew Down pressed

EA		0A = Zoom In/Out released

FA		0A = Zoom In pressed

FA		0B = Zoom Out pressed

EA		0C = Time Compress released

FA		0C = Time Compress Increment pressed

FA		0D = Time Compress Decrement pressed

FB		00 = DC Selector = Stby power

FB		01 = DC Selector = Batt Bus

FB		02 = DC Selector = Batt

FB		03 = DC Selector = TR1

FB		04 = DC Selector = TR2

FB		05 = DC Selector = TR3

FB		06 = DC Selector = Test

FB		10 = AC Selector = Stby power

FB		11 = AC Selector = Grnd power

FB		12 = AC Selector = Gen 1

FB		13 = AC Selector = APU gen

FB		14 = AC Selector = Gen 2

FB		15 = AC Selector = Inverter

FB		16 = AC Selector = Test

FB		1F = Grnd power off

FB/EB		20 = Standby power Bat

FB/EB		21 = Grnd power on/center

FB/EB		22 = Galley on/off

FB/EB		23 = Entertainment on/off

FB/EB		24 = CSD Disconnect 1

FB/EB		25 = CSD Disconnect 2

FB/EB		26 = Standby power on/off

FB/EB		27 = Engine 1A hydraulic pump on/off

FB/EB		28 = Engine 1B hydraulic pump on/off

FB/EB		29 = Engine 2A hydraulic pump on/off

FB/EB		2A = Engine 2B hydraulic pump on/off

FB/EB		2B = Yaw damper on/off

EB		2C = Engine 1 fire suppression off

FB		2C = Engine 1 fire suppression left

FB		2D = Engine 1 fire suppression right

EB		2E = Engine 2 fire suppression off

FB		2E = Engine 2 fire suppression left

FB		2F = Engine 2 fire suppression right

EB		30 = APU fire suppression off

FB		30 = APU fire supression left

FB		31 = APU fire supression right

FB/EB		32 = Fuel cutoff Eng 1 on/off

FB/EB		33 = Fuel cutoff Eng 2 on/off

EB		34 = Engine overheat test 1 = normal

FB		34 = Engine overheat test 1 = a

FB		35 = Engine overheat test 1 = b

EB		36 = Engine overheat test 2 = normal

FB		36 = Engine overheat test 2 = a

FB		37 = Engine overheat test 2 = b

FB/EB		38 = Bell cutout pressed/released

EB		39 = Fault test = off

FB		39 = Fault test = fault inop

FB		3A = Fault test = fire overheat

EB		3B = extinguish test = off

FB		3B = extinguish test = left

FB		3C = extinguish test = right

FB/EB		3D = stab trim main electric cutout/normal

FB/EB		3E = stab trim autopilot cutout/normal

FB/EB		3F = stab trim override override/normal

FB/EB		40 = landing gear horn cutout pressed/released

EB		41 = fuel flow = rate

FB		41 = fuel flow = reset

FB		42 = fuel flow = used

FB/EB		43 = left pitot heater on/off

FB/EB		44 = right pitot heater on/off

Examples:

F0 01	= COM1 active/standby button pressed

E0 01	= COM1 active/standby button released

E2 03	= COM2 coarse frequency increment

F7 07	= Transponder digit 3 decrement

*The current hardware doesn't send these, but sends transponder

control state messages (0x94, table 2) instead.

-------------------------------------------------------------

- 3. HOST-TO-PANEL MESSAGES

- Messages sent by the host to the Avionics Panel specify changes

in the panel displays. They should be sent whenever a display changes,

and occasionally resent to synchronize displays in case the

panel was disconnected or messages were lost.

- Messages are one to four bytes long; the high nibble of the first

byte is always A, B, C, or D hex (with one important exception),

while the high nibbles of any subsequent bytes are never in this

range. This allows the panel to stay synchronized with the data stream.

- The exception is the two-byte Select Protocol message: 78 FA. This

forces the Avionics Panel into native protocol mode, and should be

the first message sent by the host. It should also be sent periodically

in case communication was disrupted.

- The control panels do not support these functions.

The messages in hexadecimal are:

78 = select protocol, followed by FA (see above)

A0-A7 = marker beacon status (see Table 6)

A8 = autopilot status, followed by one-byte state (see Table 7)

A9 = landing gear status, followed by one-byte state (see Table 10)

AA = autopilot with AP light off, with one-byte state (see Table 7)

AB = relay control, with one-byte state (see Table 11)

BX = blank all displays for device X (see Table 8)

- CX = set left digit displays for device X, followed by 3 bytes

(see Table 9)

- DX = set right digit displays for device X, followed by 3 bytes

(see Table 9)

-------------------------------------------------------------

- Table 6: Marker Beacon Status

- This one-byte message consists of A0 hex plus a 3-bit code

indicating the state of the marker beacon lights (bit set =

light on, bit clear = light off):

bit 2		Airway

bit 1		Outer

bit 0 (LSB)	Middle

Example:

A2 = Outer marker beacon light on, others off

- Note: at most one bit should be set.

-------------------------------------------------------------

- Table 7: Autopilot Status

This two byte message consists of A8 hex, followed by a byte

whose bits are set to light the corresponding autpilot status LEDs:

bit 7 (MSB)	always zero

bit 6		FD

bit 5		ALT

bit 4		HDG

bit 3		BC

bit 2		GS

bit 1		NAV

bit 0 (LSB)	APR

- The AP light is always turned on when this message is received. To set

ALL autopilot LED's off, send message B6 instead.

- To set autopilot LED's *without* turning on the AP light, use a first

byte code of AA hex instead of A8.

Example:

A8 22	= Autopilot ALT and APR (and AP) lights on

-------------------------------------------------------------

- Table 8: Blank Displays

- This one-byte command causes all of the displays for the

specified device to be turned off:

B0	= Blank COM1

B1	= Blank NAV1

B2	= Blank COM2

B3	= Blank NAV2

B4	= Blank DME

B5	= Blank ADF

B6	= Blank Autopilot

B7	= Blank Transponder

BB	= Blank Jet console lights

Examples:

B1	= blank all NAV1 digit displays

B6	= turn off all Autopilot status lights

-------------------------------------------------------------

- Table 9: Set Digit Displays

- The COM, NAV, DME, and ADF devices each have ten LED digit displays

arranged as two 5-digit readouts. The Transponder has four single-digit

displays and an Ident light.

This four-byte command updates the left or right 5-digit

display, or the Transponder display, for the specified device.

- While you can use these commands to blank all the digits of a

device, the BX one-byte command (Table 7) will do that more efficiently.

- First byte

hi nibble = C (left displays or Transponder) or

D (right displays)

lo nibble = device ID (same as Table 5)

- Second byte

hi nibble = first digit (0-9=digit, E=dash, or F=blank)

lo nibble = second digit

- Third byte

hi nibble = third digit

lo nibble = fourth digit (see below regarding DME)

- Fourth byte

hi nibble = fifth digit (see notes below)

lo nibble = decimal point indicator:

8 = show decimal point after 4th digit

4 = show decimal point after 3rd digit

2 = show decimal point after 2nd digit

0 = no decimal point

Examples:

- C1 13 69 04    D1 FF 06 80

sets the NAV1 display to "136.90  __068" (where "_" = blank).

- C7 04 73 00

sets the Transponder display to "0 4 7 3".

- The decimal points and fifth digit on the ADF left-hand display are

used as status indicators:

2nd and 3rd digit decimal points = ADF

5th digit top and bottom segments = ANT

5th digit upper left and lower-right segments = BFO

The following nibble codes can be used to set the 5th digit segments:

F or E = both indicators off

2 = ANT indicator on, BFO indicator off

4 = BFO indicator on, ANT indicator off

0,5,6,8, or 9 = both indicators on

- For example, to set the left ADF display to " 030 " + ADF + BFO, send

- C5 F0 30 4C

- The fourth and fifth digits on the DME left-hand display should not

be used. Instead, the 4th digit's top and bottom segments are used for

the "NM" indicator, and its upper-left and lower-right segments are

used for the "KT" indicator. The 5th digit's top and bottom segments

are used for the "MHZ" indicator, and its upper-left and lower-right

segments are used for the "MIN" indicator.

The following nibble codes can be used to set these segments:

- F or E = both indicators off

- 2 = NM or MHZ indicator on, KT or MIN indicator off

4 = KT or MIN indicator on, NM or MHZ indicator off

0,5,6,8, or 9 = both indicators on

- For example, to set the left DME display to "12.3" + NM + KT + MIN, send

- C4 12 30 42

The Transponder uses the center segment of the fifth digit (left display)

as the Ident light. Use nibble code "E" to turn it on, "F" to turn it

off. For example, to set the Transponder display to "7654" with ident

lit, send

- C7 76 54 E0

- Table 9A: Set Jet Console Displays

The Jet console has 14 discrete indicators.

This four-byte command updates the display state.

A bit set to 1 turns an indicator on, a bit set to 0 turns it off.

While you can use this commands to blank all the displays,

the BB one-byte command (Table 7) will do that more efficiently.

- First byte

CB = jet console display

- Second byte

bit 0 = L BOTTLE DISCHARGE

bit 1 = R BOTTLE DISCHARGE

bit 2 = APU BOTTLE DISCHARGE

bit 3 = WHEEL WELL

bit 4 = FAULT

bit 5 = APU DET INOP

Third byte

bit 0 = ENG 1 OVERHEAT

bit 1 = ENG 2 OVERHEAT

Fourth byte

bit 0 = Left green

bit 1 = Right green

bit 2 = APU green

bit 3 = Eng 1 fire disch switch

bit 4 = Eng 2 fire disch switch

bit 5 = APU fire disch switch

-------------------------------------------------------------

- Table 10: Landing Gear Status

- This two byte message consists of A9 hex, followed by a byte

whose bits are set to light the corresponding landing gear status LEDs:

bit 7 (MSB)	always zero

bit 6		not used, set to zero

bit 5		Right gear in transition (red)

bit 4		Right gear down (green)

bit 3		Left gear in transition (red)

bit 2		Left gear down (green)

bit 1		Front gear in transition (red)

bit 0 (LSB)	Front gear down (green)

Example:

A9 15 = all gear down (green)

-------------------------------------------------------------

- Table 11: Relay control

This two byte message consists of AB hex, followed by a byte

turning on or off relay-controlled devices (where supported).

00 = electric pitch trim motor off

01 = electric pitch trim motor up

02 = electric pitch trim motor down

04 = stick shaker off

05 = stick shaker on

- Limit switches will automatically turn off the pitch trim motor if it

reaches the end of its travel. When a pitch trim switch is pressed

on the panel, it will automatically run the trim motor as well as

report a code to the host PC, so it's not necessary (but not harmful)

for the PC to send this message in response. However, this message

can be used to control the trim motor when, for example, the user

moves a virtual trim wheel on the screen.

Example:

AB 01 = turn pitch trim motor on in up direction

-------------------------------------------------------------