B777- 300 ER

| # Specifica di missione |  |
| --- | --- |
| ## Tipo velivolo | BIMOTORE Turbofan trasporto pass (VERY long range) ### Costruzione classica in alluminio (con 20% parti in composito) |
| Carico pagante | 400 passeggeri |
| Normativa | FAR 25 |
| Autonomia di percorso(fase 5) | 8000 nm alla quota di 35000 ft a M=0.84 Assumere E=16-18 (CDo=0.016-0.017 e=0.80) Cj =0.45-0.50 lb/lbh (stessi motori del nuovoB787) |
|  | +100 nm per aeroporto alternativo |
| Velocità massima di crociera | 0.89 Mach a 35000 ft |
| Quota di crociera | 35000 ft |
| Lunghezza di decollo fino ai 10.7 m (35 ft) (T-O field lenght) | 10500 ft at S/L |
| Lunghezza di atterraggio dai 15 m (50 ft) Al max Landing Weight (assumere MLW=0.70* MTOW) | 6500 ft at S/L |
| Rateo di salita |  |
| Riserva di carburante-attesa (fase 6) Riserva – Diversion(fase 8) | 45 minuti di attesa 100 nm a 20000 ft |
| Velivoli simili | B747-400, A380 |

1

2

4

3

6

5

9

7

Warmup

Discesa

Attesa

Crociera

Salita

Taxi

Decollo

Atterraggio

8

Aeroporto alternativo

## Boeing - 777-300ER

The 777-200LR Worldliner (Longer Range) and 777-300ER (Extended Range) are two long-range airplanes that Boeing has developed to offer airlines additional flexibility in serving the nonstop routes that passengers demand. Launched in February 2000340-600 and A340-500.

View specification units in: METRIC | IMPERIAL

## SPECIFICATION

| First Flight date: |  |  |
| --- | --- | --- |
| Certification date: | 16 Mar 2004 |  |

## PRINCIPAL DIMENSIONS

| Fuselage width: | 6.2 m |  |
| --- | --- | --- |
| Fuselage height: | 8.39 m |  |
| Fuselage length: | 73.08 m |  |
| Cabin length: | 58.5 m |  |
| Cabin width: | 5.82 m |  |
| Cabin height: |  |  |
| Cabin volume: |  |  |
| Hold volume: | 214 m3 |  |

## WEIGHTS

| Empty operating: | 167,829 kg |  |
| --- | --- | --- |
| Max zero fuel: | 237,682 kg |  |
| MTOW: | 351,533 kg |  |
| MLW: | 251,290 kg |  |
| Standard fuel capacity: | 181,264 kg |  |
| Max fuel capacity: | 181,264 kg |  |

## SPEEDS

| Normal cruise: | 895 km/h |  |
| --- | --- | --- |
| Max cruise: | 948 km/h |  |

## PERFORMANCE

| Long Range Cruise alt: | 11,278 m |  |
| --- | --- | --- |
| Max ceiling: | 13,137 m |  |
| Take off field length: | 3,191 m |  |
| Landing field length: | 1,750 m |  |
| Max Payload Range: | 10,186 m |  |
| Range notes: |  |  |
| Max passengers: | 0 |  |
| Typical passengers: | 370 |  |

## POWERPLANT

| Category: | Turbofan |  |
| --- | --- | --- |
| Manufacturer: | GE |  |
| Model/Submodel: | GE90 115B |  |
| Thrust: | 512 kN |  |